⽆⼈机防御系统

Authentication

Login 登录

Query 查询

JammerCooldownStatus 获取⼲扰器冷却状态ListSigRecords 信号记录列表PackageVersionOnline 在线获取最新软件包版本UpgradeStatusOnline 获取升级状态AirDefenceArea 防空区

AutoAttack ⽆⼈值守状态AutoAttackConfig ⽆⼈值守配置

AutocaliProgress 以百分⽐形式查询光电设备⾃动校准的进度

Blacklist ⿊名单信息Check 升级

DetectingBands 设备检测频段状态DeviceStat 设备引擎状态信息与收发包信息Devices 设备信息

Dfgun 察打枪

DhcpStatus 动态主机配置协议信息

Drone ⽆⼈机信息DroneAdsB 航班信息

DroneIDs 按⽆⼈机ID查询事件Dronetraces 历史轨迹

Dronetypes 按⽆⼈机类型查询事件Events 事件

Events aggregation 事件聚合查询Events_by_paging 事件展开查询GetSigParseImage 获取信号分析图⽚Get_gvs_conf 获取光电设备配置GfskData 通⽤检测器列表信息GfskProto 通⽤检测器协议信息LinkNodeList 组⽹节点（已移⼊列表）LocalPatchInfo 可⽤于切换的本地补丁Nodes 组⽹节点信息

Notify 通知信息

ProfileAliasList 查询⽆⼈机⾃定义机型列表ProfileList 查询⽆⼈机属性

ProfilePatternList 查询⽤于匹配⽆⼈机 ID 的模式列表Report 统计

Screenshot 图传截图Sensor 传感器SpdReport 频谱SpoofStatus 诱骗器状态Storage 设备存储空间

SysCapability 设备⽀持打击⽅式Time 查看设备系统时间TimeZone 查看设备时区UnknownSignal 未知信号

UnlinkNodeList 组⽹未知节点列表Upgrade 本地升级版本

Version 当前版本信息Vsgevent 光电单事件查询

Vsgevents 光电多事件查询Vsgeventsimage 光电图⽚查询Whitelist ⽩名单列表WidebandJammer 宽频⼲扰状态WidebandJammer2 宽频⼲扰状态Wifilist wifi列表

Mutation 操作

AddBlacklist 添加⿊名单AddWhitelist 添加⽩名单AirDefenceArea 绘制防空区Attack 精准打击

AutoAttack ⽆⼈值守ClearStorage 清除内部存储Ctrl_landing 迫降

DeleteBlacklist 移除⿊名单⽆⼈机DeleteEvents 删除事件

DeleteEventsByID 根据⽆⼈机ID删除事件DeleteWhitelist 移除⽩名单⽆⼈机DetectingBands 检测频段开关

Device 设备设置DeviceTest ⾃检测Dfgun 察打枪

DhcpEnable 开启/关闭动态主机配置服务DirectionalWBAttack 定向打击DroneAdsB 开启或关闭ADS-B

DroneProfileAlias 为指定⽆⼈机类型添加或删除别名DroneProfilePattern 添加或删除⽆⼈机正则表达式模式DroneSub 订阅⽆⼈机截图信息（图传）

Drone_profile 修改⽆⼈机属性Gfsk 通⽤检测器

GvsControl 控制光电服务（⾃动校准服务）MarkFalseAlarm 标记误报（光电）MarkFalseAlarmByID 按ID标记误报（光电）Node 节点

PowerMode 设置电源模式，仅适⽤于便携式设备Remove_fake_drone 标记⽆⼈机为误报⽆⼈机（⽆⼈机列表）Reset 重启设备

RollbackPatch 切换或升级 UI 版本SetGeoLocation 设置设备位置（⾮持久化）Set_gvs_conf 修改光电配置Set_sensor_mode 设置传感器模式Set_vsg_license 设置光电授权

SigParse 信号分析signalCap 信号抓取SignalRec 信号证据记录Spd 频谱

Spoofer_switch 诱骗设置StageReset 定向复位Tdoa_track TDOA跟踪Time 设置设备系统时间TimeZone 设置设备时区Tracking ⽆⼈机视觉跟踪UnknownWifi 未知WIFI

UpgradePatch 补丁升级UpgradeTo 版本升级VersionPackage 版本包Wideband_attack 宽频⼲扰WifiBgLearning wifi背景学习开关

WithdrawFalseAlarm 还原误报事件（光电）

Subscription 订阅Drone ⽆⼈机Message 后台消息Notify 通知消息

⽆⼈机防御系统Base URLs:

https://192.168.100.100/rf/graphql Authentication Login 登录

Method：POST

Request example

{

"username": "user", "password": "password"

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| username | String | ⽤⼾名 |
| password | String | 密码 |

Responses

| HTTP Status Code | Meaning | Description | Data schema |
| --- | --- | --- | --- |
| 200 | OK | 成功（后续接⼝需⽤token） | Inline |

Response Examples

Success

{

"token": "eyJhbGciOiJIUzI1NiIcCI6IkpXVCJ9.eyJpZCI6ImQ120NjRmMDJjLTRlOTUmZiLTY3ZTE4NjBlZjA0MiImlhdCI6M TcwOTE5MTg3M1CwiZXjoxNzQwNzI3ODcwfQ.7eHCVT6-Prm2VHorbCN4zkf5u_vEViHXHsDec91is4",

"expLen": 31536000000,

"user": {

"id": "d464f02c-4e95-11ec-b2fb-67e1860ef042", "displayName": "root",

"imageUrl": "https://s3.amazonaws.com/uifaces/faces/twitter/lepinski/128.jpg", "role": "root",

"create_user_maxnum": null, "creator": null, "controller2sites": {}, "controllers": {},

"sites": {}, "controller": {}

}

}

Failure

"Unauthorized"

Query 查询

JammerCooldownStatus 获取⼲扰器冷却状态

Method：POST

Request example

query

{

JammerCooldownStatus {

band12 band14 band15 band18 band24 band4 band58 band9

}

}

Parameter Description

Responses

| NameRequest example | Type | Description |
| --- | --- | --- |
| band12 | Boolean | 1.2G频段的冷却状态 |
| band14 | Boolean | 1.4G频段的冷却状态 |
| band15 | Boolean | 1.5G频段的冷却状态 |
| band18 | Boolean | 1.8G频段的冷却状态 |
| band24 | Boolean | 2.4G频段的冷却状态 |
| band4 | Boolean | 433M波段的冷却状态 |
| band58 | Boolean | 5.8G频段的冷却状态 |
| band9 | Boolean | 900M频段的冷却状态 |

Boolean

Response Examples

Success

{

"data": {

"JammerCooldownStatus": { "band12": null, "band14": null, "band15": true, "band18": null, "band24": true,

"band4": null, "band58": true, "band9": true

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"JammerCooldownStatusd\" on type \"Query\". Did you

mean \"JammerCooldownStatus\"?",

"locations": [

{

"line": 3, "column": 3

}

]

}

]

}

ListSigRecords 信号记录列表

Method：POST

Request example

query{ ListSigRecords (

offset:0

limit:1

)

{

id

size

updated_time

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| offset | Int | 查询记录的偏移量 |
| limit | Int | 每⻚记录数，默认10 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| id | String | 信号记录ID |
| size | Int | 记录⽂件⼤⼩ |
| updated_time | DateTime | 证据记录时间 |

Response Examples

Success

{

"data": {

"ListSigRecords": [

{

"id": "81",

"size": 6881392,

"updated_time": "2025-07-01T19:10:44.94+08:00"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"siz\" on type \"SigRecordInfo\". Did you mean

\"size\"?",

"locations": [

{

"line": 9, "column": 5

}

]

}

]

}

PackageVersionOnline 在线获取最新软件包版本

Method：POST

Request example

query

{

PackageVersionOnline

}

Parameter Description

None

Response Examples

Success

{

"data": {

"PackageVersionOnline": "v3.15.0"

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"PackageVersionOnlined\" on type \"Query\". Did

you mean \"PackageVersionOnline\"?",

"locations": [

{

"line": 3, "column": 3

}

]

}

]

}

UpgradeStatusOnline 获取升级状态

Method：POST

Request example

query

{

UpgradeStatusOnline

}

Parameter Description

None

Response Examples

Success

{

"data": {

"UpgradeStatusOnline": "UpgradeStatusIdle"

}

}

进度描述：

“UpgradeStatusIdle”：升级尚未开始或已完成；“UpgradeStatusDownloading”：正在下载升级文件；“UpgradeStatusDownloadErr”：下载升级文件时出错；“UpgradeStatusUnpacking”：正在解压缩升级文件；“UpgradeStatusUnpackErr”：解压缩升级文件时出错；“UpgradeStatusDoUpgradeing”：正在执行升级过程；“UpgradeStatusDoUpgradeErr”：升级过程中出错；“UpgradeStatusRestarting”：升级完成后，系统将重新启动。

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"UpgradeStatusOnlined\" on type \"Query\". Did you

mean \"UpgradeStatusOnline\"?",

"locations": [

{

"line": 2, "column": 3

}

]

}

]

}

AirDefenceArea 防空区

Method：POST

Request example

query

{

airDefenceArea{

data

}

}

Parameter Description

Responses

| NameRequest example | Type | Description |
| --- | --- | --- |
| data | jsonStrType | 防控区域 |

Response Examples

Success

{

"data": {

"airDefenceArea": [

{

"data": "{\"type\":1,\"areatype\":0,\"locations\":

[[116.370964,40.010919]],\"distance\":1401.62623426214,\"hash\":\"2397271306\"}"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"date\" on type \"areaDefenceType\". Did you

mean \"data\"?",

"locations": [

{

"line": 4, "column": 5

}

]

}

]

}

AutoAttack ⽆⼈值守状态

Method：POST

Request example

query

{

autoAttack

}

Parameter Description

None

Response Examples

Success

{

"data": {

"autoAttack": "false"

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"auto\" on type \"Query\".", "locations": [

{

"line": 3, "column": 3

}

]

},

{

"message": "Cannot query field \"Attack\" on type \"Query\". Did you mean

\"autoAttack\"?",

"locations": [

{

"line": 3, "column": 8

}

]

}

]

}

AutoAttackConfig ⽆⼈值守配置

Method：POST

Request example

query

{

autoAttackConfig {

dirWbEnabled on protectWhite saEnabled spfEnabled wbEnabled

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| dirWbEnabled | Boolean | 是否开启定向打击 |
| on | Boolean | ⽆⼈值守开关 |
| protectWhite | Boolean | 是否开启⽩名单保护 |
| saEnabled | Boolean | 是否开启⾃动打击 |
| spfEnabled | Boolean | 是否开启诱骗 |
| wbEnabled | Boolean | 是否开启宽频打击 |

Response Examples

Success

{

"data": {

"autoAttackConfig": { "dirWbEnabled": true, "on": false, "protectWhite": false, "saEnabled": true, "spfEnabled": false, "wbEnabled": true

}

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"one\" on type \"AutoAttackConfig\". Did you

mean \"on\"?",

"locations": [

{

"line": 5,

"column": 5

}

]

}

]

}

AutocaliProgress 以百分⽐形式查询光电设备⾃动校准的进度

Method：POST

Request example

query

{

autocaliProgress {

details errorCode percentage

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| details | Int | ⾃动协调进度代码详情 0:⾃动协调停⽌ 1:⾃动协调开始 2:完成 3:错误 |
| errorCode | Int | autocali 错误 0:正常 1:创建错误 2:写⼊错误 3:读取错误 4:approch ptz 错误 5.get ptz 错误 6:捕获错误 7:数学计算失败 8:连接错误 9:取消 10:超时 |
| percentage | Int | 进度百分⽐ 0~100 |

Response Examples

Success

{

"data": {

"autocaliProgress": { "details": 0, "errorCode": 0, "percentage": 0

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"detail\" on type \"ProgressMsgType\". Did you

mean \"details\"?",

"locations": [

{

"line": 3,

"column": 5

}

]

}

]

}

Blacklist ⿊名单信息

Method：POST

Request example

query

{

blacklist { alias description dronetype

id

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| alias | String | ⽆⼈机别名 |
| description | String | ⽆⼈机描述 |
| dronetype | String | ⽆⼈机类型 |
| id | String | ⽆⼈机ID |

Response Examples

Success

{

"data": {

"blacklist": [

{

"alias": "", "description": "",

"dronetype": "CFLY General",

"id": "a09f10664c66"

},

{

"alias": "",

"description": "",

"dronetype": "POWERVISION PowerEggX", "id": "e09153a9336b"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"i\" on type \"blacklist\". Did you mean

\"id\"?",

"locations": [

{

"line": 6, "column": 5

}

]

}

]

}

Check 升级Method：POST Request example

query

{

check (pkg_size:1.111) {

free_space has_enough_space local_versions {

version_id version_publish_time version_status

} prepare_status upgrade_status

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| pkg_size | Float | 版本包⼤⼩ |

Responses

| Name | Type | Description |
| --- | --- | --- |
| free_space | Int | ⽤于升级的可⽤空间（单位：Mbytes） |
| has_enough_space | Boolean | 是否有⾜够的空间进⾏升级 |
| local_versions | List | 有效的本地版本 |
| version_id | String | 版本的唯一 ID |
| version_publish_time | String | 发布时间 |
| version_status | String | 版本状态和有效性 |
| prepare_status | String | 准备升级⽬录 |
| upgrade_status | String | 升级状态 |

Response Examples

Success

{

"data": {

"check": {

"free_space": 93, "has_enough_space": true, "local_versions": [

{

"version_id": "v3.13.0",

"version_publish_time": "2024-06-27T12:25:17Z", "version_status": "Valid"

}

],

"prepare_status": "done",

"upgrade_status": "updated"

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"prepare_statuse\" on type \"upgradeType\". Did you

mean \"prepare_status\" or \"upgrade_status\"?",

"locations": [

{

"line": 13,

"column": 5

}

]

}

]

}

DetectingBands 设备检测频段状态

Method：POST

Request example

query{

detectingBands {

Boost24 band12 band14 band18 band24 band4 band58 band9 bandext

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| Boost24 | Int | 测试功能、 0:不可⽤，1:禁⽤，2:启⽤。 |
| band12 | Int | 1.2Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band14 | Int | 1.25hz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band18 | Int | 1.8Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band24 | Int | 2.4Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band4 | Int | 433Mhz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band58 | Int | 5.8Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band9 | Int | 900Mhz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |

| Name | Type | Description |
| --- | --- | --- |
| bandext | Int | 扩展频段状态，0:不可⽤，1:禁⽤，2:启⽤。 |

Response Examples

Success

{

"data": {

"detectingBands": { "Boost24": 0, "band12": 0, "band14": 2, "band18": 0, "band24": 2, "band4": 2, "band58": 2, "band9": 2, "bandext": 2

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"band90\" on type \"DetectingBands\". Did you

mean \"band14\", \"band9\", \"band12\", \"band4\", or \"band24\"?",

"locations": [

{

"line": 11, "column": 5

}

]

}

]

}

DeviceStat 设备引擎状态信息与收发包信息

Method：POST

Request example

query

{

deviceStat (id:"0") {

id

class pkt_stat run_stat

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | 设备引擎ID |

Responses

| Name | Type | Description |
| --- | --- | --- |
| class | Int | 设备名称 |
| id | Int | 设备ID |
| pkt_stat | Int | 基于每个传感器的引擎数据包统计 |
| run_stat | Int | 引擎的运⾏状态 |

Response Examples

Success

{

"data": {

"deviceStat": [

{

"class": "engine",

"id": "4418",

"pkt_stat": "{\"1\":{\"DETECT\":{\"1.4G\": {\"alert_rate\":0,\"pkt_rate\":0,\"tot_alerts\":0,\"tot_pkts\":2524},\"2.4G\": {\"alert_rate\":0,\"pkt_rate\":0,\"tot_alerts\":1,\"tot_pkts\":2028},\"5G\": {\"alert_rate\":0,\"pkt_rate\":0,\"tot_alerts\":6,\"tot_pkts\":12643},\"Sub-1G\": {\"alert_rate\":0,\"pkt_rate\":0,\"tot_alerts\":0,\"tot_pkts\":5074},\"Unknown\": {\"alert_rate\":0,\"pkt_rate\":0,\"tot_alerts\":8,\"tot_pkts\":1514},\"all\": {\"alert_rate\":0,\"pkt_rate\":0,\"tot_alerts\":15,\"tot_pkts\":23783}}}}",

"run_stat": " {\"cpu_usage\":22,\"cpu_freq\":1412,\"cpu_temp\":39,\"cpu_ram_used\":6534,\"cpu_ram_total\": 7765,\"gpu_usage\":0,\"gpu_temp\":38,\"gpu_ram_used\":6534,\"gpu_ram_total\":7765,\"swap_use d\":1524,\"swap_total\":3883}"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Argument \"id\" has invalid value 0.\nExpected type \"String\",

found 0.",

"locations": [

{

"line": 4, "column": 9

}

]

}

]

}

Devices 设备信息

Method：POST

Request example

query

{

devices(id:"0"){

class config faults

gps_fixed

history { dev_id dev_type

time_of_connection

time_of_disconnection

}

id

node rec_status scv_status sigCap_status state

status test_status toc

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | 设备id，不传显⽰所有 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| class | String | 当前设备名称 |
| config | jsonStrTypele | 当前设备配置 |
| faults | String | 当前设备故障信息 |
| gps_fixed | Boolean | GPS固定（false未固定-按钮关闭，true固定-按钮开启） |
| history | String | 连接丢失历史 |
| dev_id | String | 设备ID |
| dev_type | String | 设备类型 |
| time_of_connection | String | 连接时间 |
| time_of_disconnection | String | 断开时间 |
| id | String | 设备ID |
| node | String | 设备所属节点 |
| rec_status | String | 设备记录状态 |

| Name | Type | Description |
| --- | --- | --- |
| scv_status | jsonStrType | 传感器_收发包状态 |
| sigCap_status | String | 捕捉引擎设备状态的信号 |
| state | String | 当前设备的当前状态:停⽌、运⾏、降级、更新（各个设备的运⾏状态） |
| status | String | 当前设备状态信息（如：控制器状态、传感器状态等） |
| test_status | String | 当前设备测试状态_测试模式是否开启 |
| toc | String | 连接时间_持续时间段（时间戳换算-毫秒级） |

Response Examples

Success

{

"data": {

"devices": [

{

"class": "sensor",

"config": "{\"version\":\"2024.03.19\",\"build_time\":\"2024-03- 19T01:14:30Z\",\"git_hash\":\"3a79c31c4f3f9698dad3890f9b3f969b733af8be\",\"device_type\":\"D etectionOnly\",\"mqtts_ip\":\"192.178.1.88\",\"mqtts_hostname\":\"1.dcn.local\",\"id\":\"1\" ,\"group\":\"AT\",\"mac\":\"00:0a:35:00:01:22\",\"sn\":\"SMMMMMMMM0440001R000\",\"pid\":\"DF T-3000AD\",\"hardware_version\":\"2\",\"rfchain_master\":true,\"ttl\":15,\"geo_location\": {\"lat\":40.02718,\"lng\":116.0711},\"location_locked\":true,\"range\":8000,\"mimo_num\":1,\ "eng_port\":\"8000\",\"eng_ip\":\"192.178.1.88\",\"n_eng\":2,\"listen_wifi\":\"\",\"watchdog _enable\":false,\"compass_enable\":true,\"dfant_id\":\"\",\"crif_califile\":\"crif_cali.bin\ ",\"is_sdexist\":true,\"cps_offset\":0,\"bands\": [\"band14\",\"band9\",\"band4\",\"band58\",\"band24\"],\"with_spf\":false}",

"faults": "", "gps_fixed": false, "history": null, "id": "1",

"node": "master", "rec_status": "",

"scv_status": "{\"version\":\"2024.03.19\",\"built_time\":\"2024-03- 19T01:15:06Z\",\"git_hash\":\"5f9136d55dba3274d2a00d93caa2d771b21859a9\",\"temperatures\": [47368,69843],\"tot_pkts\":[148135,1],\"pkts_rate\":[4.16,0]}",

"sigCap_status": "", "state": "operational",

"status": "{\"ip_address\":\"192.178.1.251/24\",\"first_seen\":\"25 Mar 24 15:25 CST\",\"last_seen\":\"\",\"version\":\"2024.03.19\",\"built_time\":\"2024-03- 19T01:14:30Z\",\"git_hash\":\"3a79c31c4f3f9698dad3890f9b3f969b733af8be\",\"protected\":false ,\"spd_cap\":true,\"spd_running\":false,\"heading\":-1,\"heading_reset\":false,\"gps_status\ ":\", 0.00\",\"test_gen\":0}",

"test_status": "{\"result\":0,\"test_cases\":null,\"time\":\"0001-01-

01T00:00:00Z\"}",

"toc": "70364329"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"tocc\" on type \"Devices\". Did you mean

\"toc\"?",

"locations": [

{

"line": 22, "column": 5

}

]

}

]

}

Dfgun 察打枪Method：POST Request example

query

{

dfgun { attack_freq_high attack_freq_low dBm

droneid freq heading mode rssi

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| attack_freq_high | Int | 可调功率放⼤器（⾼） |
| attack_freq_low | Int | 可调功率放⼤器（低） |

| Name | Type | Description |
| --- | --- | --- |
| dBm | Int | 分⻉毫⽡ |
| droneid | Int | ⽆⼈机ID |
| freq | String | 频率 |
| heading | Float | ⽅向 |
| mode | Int | 运⾏模式 |
| rssi | Int | 信号强度0~100 |

Response Examples

Success

{

"data": {

"dfgun": { "attack_freq_high": 0, "attack_freq_low": 0, "dBm": 0,

"droneid": "", "freq": 0, "heading": -1, "mode": 0, "rssi": 0

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"rss\" on type \"dfgunStatus\". Did you mean

\"rssi\"?",

"locations": [

{

"line": 11, "column": 5

}

]

}

]

}

DhcpStatus 动态主机配置协议信息

Method：POST

Request example

query

{

dhcpStatus {

gateway ip netmask on status

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| gateway | String | ⽹关 |
| ip | string | IP地址 |
| netmask | String | ⼦⽹掩码 |
| on | Boolean | DHCP on/off，如果这个是off，所有其他字段都应该被忽略 |
| status | Int | DHCP状态：0--未连接，1--已连接 |

Response Examples

Success

{

"data": {

"dhcpStatus": {

"gateway": "192.168.85.1", "ip": "192.168.85.45", "netmask": "255.255.254.0", "on": true,

"status": 1

}

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"ip1\" on type \"dhcp\". Did you mean \"ip\"?", "locations": [

{

"line": 4, "column": 5

}

]

}

]

}

Drone ⽆⼈机信息

Method：POST

Request example

query

{

drone (id:"NAO921511865") {

altitude attack_bands attack_type attacking attacking_ttl blacklisted can_attack can_ctrl_landing can_takeover confirmed created_time ctrl_landing current_ptz {

pan tilt zoom

}

custom_name deleted_time description direction

directional_attack_state

distance has_duplicate has_screenshot height

id image in_ada

initial_location {

lat

lng

} jamming_conflicts lastseen lastseen_time latitude

link_id

localization {

lat

lng

} longitude name

rc_distance

rc_location {

lat

lng

} reliability screenshot

seen_sensor { bandwidth_khz detected_freq_khz noise_dbm

port sensor_id signal_dbm snr_dB

} speed state

tracing (from:0) {

lastlen

origin {

lat

lng } points

} tracking_video whitelisted

}

}

Parameter Description

Requests

| Name | Type | Descripti |
| --- | --- | --- |
| id | String | 检测到的⽆⼈机id |

Responses

| Name | Type | Descripti |
| --- | --- | --- |
| altitude | Float | 海拔 |
| attack_bands | Int | ⽆⼈机信号 |
| attack_type | String | ⽆⼈机打击类型（Wideband宽频、Smart_CRPC打击、Smart-II精准打击、null） |
| attacking | Boolean | 是否在精准打击（true是，false否） |
| attacking_ttl | Int | 精准打击时⻓（倒计时） |
| blacklisted | Boolean | ⽆⼈机是否被列⼊⿊名单 |
| can_attack | Boolean | ⽆⼈机能否被精准打击（true、false） |
| can_ctrl_landing | Boolean | ⽆⼈机能否受控迫降 |
| can_takeover | Boolean | ⽆⼈机是否可以被接管 |
| confirmed | Boolean | 确定是⽆⼈机或只是可疑 |
| created_time | DateTime | ⾸次检测到的时间 |
| ctrl_landing | Boolean | ⽆⼈机正在受控降落 |
| current_ptz { pan tilt zoom } | CurrentPtz | ⽆⼈机最后一次看到的实时PTZ{⽅位⻆ ⾼度 俯仰⻆} |
| custom_name | String | ⽆⼈机⾃定义名称（来⾃⿊名单或⽩名单） |
| deleted_time | DateTime | 消失时间 |
| description | String | ⽆⼈机描述 |
| detectors | [String] | 检测来源列表[RID、CRPC、DroneID] |
| direction | Float | ⽆⼈机⻆度（365表⽰⽆效） |
| directional_attack_state | Int | 定向打击状态（0空闲、1追踪、2攻击） |
| distance | Float | ⽆⼈机到设备中⼼点的距离 |
| has_duplicate | Boolean | ⽆⼈机是否隐藏 |
| has_screenshot | Boolean | 图传 |

| Name | Type | Descripti |
| --- | --- | --- |
| height | Float | ⽆⼈机⾼度 |
| id | String | ⽆⼈机ID |
| image | String | ⽆⼈机图⽚ |
| in_ada | Int | ⽆⼈机在防空区（0不可⽤、1不在、2预警区、3防御区） |
| initial_location{lat,lng} | String | ⽆⼈机初始位置，210表⽰⽆效 |
| jamming_conflicts | String | 误打列表 |
| lastseen | String | 最后发现的时间 |
| lastseen_time | DateTime | 最后一次检测到的时间 |
| latitude | String | ⽆⼈机的纬度，如果 “⽅向 ”⽆效，则⽆效 |
| link_id | Float | ⽆⼈机关联ID |
| localization{lat,lng} | {Float} | 对⽆⼈机进⾏跟踪（已弃⽤） |
| longitude | Float | ⽆⼈机的经度，如果 “⽅向 ”⽆效，则⽆效 |
| name | String | ⽆⼈机机型或别名 |
| rc_distance | Float | ⽆⼈机⻜⾏员距离，“-1”表⽰⽆效 |
| rc_location{lat,lng} | GeoPoint | ⻜⼿（遥控器）位置（210表⽰⽆效）{经纬度} |
| reliability | Int | ⽆⼈机可信度 |
| screenshot | String | ⽆⼈机最新截图（JPG格式） |
| seen_sensor | SignalObject | 传感器检测到⽆⼈机信息 |
| bandwidth_khz | Int | 信号带宽 |
| detected_freq_khz | Int | 信号频点 |
| noise_dbm | Int | 天线的射频噪声功率，分⻉差为一毫⽡ |
| port | String | 检测端⼝ |
| sensor_id | String | 检测的传感器ID |
| signal_dbm | Int | 信号⼤⼩ |
| snr_dB | Float | 信噪⽐ |
| speed | Float | ⽆⼈机速度（m/s） |
| state | String | ⽆⼈机状态（attacking打击、danger危险、Smart_打击类型） |
| tracing{lastlen} | String | ⽆⼈机轨迹点 |

| Name | Type | Descripti |
| --- | --- | --- |
| from | Int | 从 |
| lastlen | Int | 最后一个点的⻓度 |
| origin{lat,lng} | Float | ⽆⼈机轨迹的参考中⼼点位置 |
| points | [Float] | ⽆⼈机轨迹点 |
| tracking_video | String | 轨迹视频⽂件 |
| whitelisted | Boolean | 是否加⼊⽩名单 |

Response Examples

Success

{

"data": {

"drone": [

{

"altitude": 99,

"attack_bands": [

900, 2400, 5800

],

"attack_type": "null", "attacking": false, "attacking_ttl": 0, "blacklisted": true, "can_attack": false, "can_ctrl_landing": false, "can_takeover": false, "confirmed": true,

"created_time": "2024-10-17T14:27:23.154497991+08:00", "ctrl_landing": false,

"deleted_time": "0001-01-01T00:00:00Z", "description": "",

"direction": 258.94598, "directional_attack_state": 0, "distance": 10101, "has_duplicate": false, "has_screenshot": false, "height": -260,

"id": "NAO921511865",

"image": "dimages/autel_nano.png",

"in_ada": 1, "initial_location": {

"lat": 40.02487469946916, "lng": 116.3570900976692

},

"jamming_conflicts": [],

"lastseen": null,

"lastseen_time": "2024-10-17T14:27:44.133071015+08:00", "latitude": 40.02487469946916,

"link_id": "", "localization": null,

"longitude": 116.3570900976692, "name": "AUTEL EVO-Nano+", "rc_location": {

"lat": 210, "lng": 210

},

"screenshot": "",

"seen_sensor": [

{

"bandwidth_khz": 10000, "detected_freq_khz": 2412000, "noise_dbm": 0,

"port": "rf", "sensor_id": "2", "signal_dbm": 0, "snr_dB": 0

}

],

"speed": 20, "state": "safe", "tracing": {

"lastlen": 1121, "origin": {

"lat": 40.0423, "lng": 116.4736

},

"points": [

[

-9048, -134.2,

0

],

[

-9914.4, -1936.8,

0

],

[

-9914.4,

-1936.8, 0

],

……

]

},

"tracking_video": "",

"whitelisted": false

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"attack_band\" on type \"Drone\". Did you mean

\"attack_type\", \"attack_bands\", or \"attacking\"?",

"locations": [

{

"line": 5, "column": 5

}

]

}

]

}

DroneAdsB 航班信息

Method：POST

Request example

query

{

droneAdsB {

drone

{

confirmed detected_freq_khz height

id latitude longitude

manufacturer

model

owner registration speed typecode yaw_angle

} support switch

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| drone | List | ⽆⼈机列表 |
| confirmed | Boolean | ⽆⼈机状态 |
| detected_freq_khz | Int | ADS-B频数 |
| height | Float | ⻜机⾼度 |
| id | String | ⻜机id |
| latitude | Float | ⻜机纬度 |
| longitude | Float | ⻜机经度 |
| manufacturer | String | ⻜机制造商 |
| model | String | ⻜机模型 |
| owner | String | ⻜机拥有者 |
| registration | String | ⻜机登记 |
| speed | Float | ⻜机速度 |
| typecode | String | ⻜机类型的代码 |
| yaw_angle | Float | ⻜机偏航⻆ |
| support | Boolean | 是否⽀持 |
| switch | Boolean | 是否开启状态 |

Response Examples

Success

{

"data": {

"droneAdsB": {

"drone": [

{

"confirmed": true, "detected_freq_khz": 1090000, "height": 0,

"id": "26a59e",

"latitude": 39.63443562135858, "longitude": 114.6409912109375, "manufacturer": "",

"model": "", "owner": "",

"registration": "", "speed": 0, "typecode": "", "yaw_angle": 70.928375

}

],

"support": true, "switch": true

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"confirme\" on type \"DroneAdsB\". Did you mean

\"confirmed\"?",

"locations": [

{

"line": 6, "column": 7

}

]

}

]

}

DroneIDs 按⽆⼈机ID查询事件

Method：POST

Request example

query

{

droneIDs

(

begin_time:"2024-10-01T09:47:09.590479Z" end_time:"2024-10-17T23:15:14.565626Z" pattern:""

limit:5

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| begin_time | String | 查询开始时间 |
| end_time | String | 查询结束时间 |
| pattern | String | 模糊查询匹配字段 |
| limit | Int | 最⼤记录数 |

Response Examples

Success

{

"data": {

"droneIDs": [ "000800102b95", "0008001042a6", "00080010634c", "000800106bcc", "0008001070bb"

]

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"limi\" on field \"droneIDs\" of type \"Query\".

Did you mean \"limit\"?",

"locations": [

{

"line": 8,

"column": 5

}

]

}

]

}

Dronetraces 历史轨迹

Method：POST

Request example

query

{

dronetraces

(

from:0 to:30400

)

{

binding_id center_lat center_lng created_time drone_type frequence

id points zoom

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| from | Int | 传事件序列号（开始） |
| to | Int | 传事件序列号（结束） |

Responses

| Name | Type | Description |
| --- | --- | --- |
| query | String | None |
| binding_id | 地图默认缩放级别 | 事件序列号 |
| center_lat | String | 传感器中⼼纬度 |
| center_lng | String | 传感器中⼼经度 |
| created_time | DateTime | 创建时间 |
| drone_type | String | ⽆⼈机类型 |
| frequence | String | 频率 |
| id | String | ⽆⼈机唯一ID |
| points | String | 根据中⼼跟踪点 |
| zoom | Int | 地图默认缩放级别 |

Response Examples

Success

{

"data": {

"dronetraces": [

{

"binding_id": 1057, "center_lat": "40.04114", "center_lng": "116.36742",

"created_time": "2025-07-01T08:44:09.411632Z",

"drone_type": "Skydio 2+SDR35", "frequence": "5745.0Mhz",

"id": "1668BS20JB00T326",

"points": "[[-20341050.6,996285.7,80,1]]", "zoom": 13

}，……

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"binding_i\" on type \"DroneTrace\". Did you mean

\"binding_id\"?",

"locations": [

{

"line": 9, "column": 5

}

]

}

]

}

Dronetypes 按⽆⼈机类型查询事件

Method：POST

Request example

query

{

dronetypes

(

limit:5 begin_time:"2024-03-19T09:47:09.590479Z" end_time:"2024-03-18T23:15:14.565626Z" pattern:"w"

all:false

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| limit | Int | 最⼤记录数（每⻚展⽰数量） |
| begin_time | String | 查询开始时间 |
| end_time | String | 查询结束时间 |
| pattern | String | 模糊搜索匹配字段 |
| all | Boolean | 如果为 true，搜索所有类型 |

Response Examples

Success

{

"data": {

"dronetypes": [

"WFLY Wft09sii",

"DJI Air2S", "DJI Mavic3",

"DJI MavicAir2", "FEIPAI 6KPro"

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Argument \"all\" has invalid value fals.\nExpected type \"Boolean\",

found fals.",

"locations": [

{

"line": 9, "column": 9

}

]

}

]

}

Events 事件Method：POST Request example

query

{

events

(

drone_id:"F45TB22BF00E021U" drone_type:"DJI Mavic3" begin_time:"2024-03-15 09:44:00" end_time:"2024-03-18 09:44:59" from:7145

)

{

attacked blacklisted created_time deleted_time drone_type first_pos frequence

id

image is_false_alarm last_pos

rc_pos seen_sensors sequence severity siteLatitude siteLongitude whitelisted

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| drone_id | String | 查询的⽆⼈机ID |
| drone_type | String | 查询的⽆⼈机类型 |
| begin_time | String | 查询开始时间 |
| end_time | String | 查询结束时间 |
| from | Int | 传事件序列号开始 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| attacked | Boolean | 是否受到攻击 |
| blacklisted | DateTime | 是否为⿊名单 |
| created_time | DateTime | 创建时间 |
| deleted_time | DateTime | 删除时间 |
| drone_type | String | ⽆⼈机类型 |
| first_pos | String | ⾸次发现位置 |
| frequence | String | 频率 |
| id | String | ⽆⼈机唯一ID |
| image | String | ⽆⼈机图像 |
| is_false_alarm | Boolean | 是否标记为误报 |
| last_pos | String | 最后发现位置 |
| rc_pos | String | ⽆⼈机⻜⼿位置 |
| reliability | Int | 该历史事件可信度，0 表⽰⽆效 |
| seen_sensors | String | 由哪个传感器看到的 |
| sequence | String | 事件序列号 |
| severity | Int | 严重程度：0：不可⽤（未知），1：低 2：中 3：⾼ |
| siteLatitude | Float | 站点纬度，210--表⽰⽆效 |
| siteLongitude | Float | 站点经度，210--表⽰⽆效 |
| whitelisted | Boolean | 是否加⼊⽩名单 |

Response Examples

Success

{

"data": {

"events": [

{

"attacked": false,

"blacklisted": false,

"created_time": "2025-06-10T06:24:33.365836Z", "deleted_time": "2025-06-10T06:24:53.366071Z", "drone_type": "DJI MavicAir2",

"first_pos": "", "frequence": "5830.5Mhz", "id": "1WNBH5B00200L5",

"image": "dimages/mavic_air2.png", "is_false_alarm": false, "last_pos": "",

"rc_pos": "", "seen_sensors": "1", "sequence": 11833, "severity": "0", "siteLatitude": 45.750248, "siteLongitude": 126.63864, "whitelisted": false

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"whiteliste\" on type \"Event\". Did you mean

\"whitelisted\"?",

"locations": [

{

"line": 28, "column": 5

}

]

}

]

}

Events aggregation 事件聚合查询

Method：POST

Request example

query

{

events_aggregation

(

offset:0 limit:1

drone_type:"CYRF6936 General" drone_id:"3991062614" begin_time:"2024-03-01T00:57:53.564197Z" end_time:"2024-12-19T23:57:53.564197Z" filter_attacked:false

filter_severity:[SEVERITY_NA,SEVERITY_LOW,SEVERITY_HIGH,SEVERITY_MEDIUM] filter_has_rc_pos:false

filter_has_pos:false filter_has_trace:false order:"desc" order_by:"created_time" data_num_limit:false

)

{

count

data

{

count

data

{

attacked blacklisted created_time deleted_time drone_type first_pos frequence has_screenshot id

image is_false_alarm last_pos

rc_pos reliability seen_sensors sequence severity siteLatitude siteLongitude whitelisted

}

}

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| drone_type | String | 按⽆⼈机类型筛选 |
| data_num_limit | Boolean | 是否限制返回数据项的数量 |
| limit | Int | 每⻚记录数，默认10 |
| order_by | String | 按此⽂件排序，值：created_time：检测时间，duration：⽆⼈机持续时间，drone_id：⽆⼈机 ID，drone_type：⽆⼈机类型，count：⽆⼈机出现次数。 默认值：created_time |
| offset | Int | 从该⽂件查询 |
| filter_attacked | Boolean | 筛选是否被⼲扰历史事件 |
| filter_severity | [Severity] | 筛选⽆⼈机危险等级历史事件SEVERITY_NA：未知等级；SEVERITY_LOW：低危险等级；SEVERITY_MEDIUM：中危险等级；SEVERITY_HIGH：⾼危险等级。 |
| filter_has_rc_pos | Boolean | 筛选是否有⻜⼿位置历史事件 |
| filter_has_pos | Boolean | 筛选是否有⽆⼈机位置历史事件 |
| filter_has_trace | Boolean | 筛选是否有轨迹历史事件 |
| drone_id | String | 按⽆⼈机id筛选 |
| begin_time | String | 查询开始时间 |
| end_time | String | 查询结束时间 |
| order | String | 数据顺序，asc升序,desc降序，默认降序 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| count | Int | 记录数 |
| data | List | EventPaging_事件分⻚ |
| count | Int | 记录数 |
| data | List | Event_⽆⼈机历史事件 |
| attacked | Boolean | 是否受到攻击 |
| blacklisted | Boolean | 是否为⿊名单 |
| created_time | DateTime | 创建时间 |
| deleted_time | DateTime | 删除时间 |

| Name | Type | Description |
| --- | --- | --- |
| drone_type | String | ⽆⼈机类型 |
| first_pos | String | ⾸次发现位置 |
| frequence | String | 频率 |
| has_screenshot | Boolean | 是否有截图 |
| id | String | ⽆⼈机唯一ID |
| image | String | ⽆⼈机图像 |
| is_false_alarm | Boolean | 是否标记为误报 |
| last_pos | String | 最后发现位置 |
| rc_pos | String | ⽆⼈机⻜⼿位置 |
| reliability | Int | 该历史事件可信度，0 表⽰⽆效 |
| seen_sensors | String | 发现⽆⼈机的传感器ID |
| sequence | String | 事件序列号 |
| severity | Int | 严重程度：0：不可⽤（未知），1：低 2：中 3：⾼ |
| siteLatitude | Float | 站点纬度，210--表⽰⽆效 |
| siteLongItude | Float | 站点经度，210--表⽰⽆效 |
| whitelisted | Boolean | 是否加⼊⽩名单 |

Response Examples

Success

{

"data": {

"events_aggregation": {

"count": 1, "data": [

{

"count": 3, "data": [

{

"attacked": false,

"created_time": "2024-10-17T06:52:12.953315Z", "deleted_time": "2024-10-17T06:52:22.914674Z", "drone_type": "CYRF6936 General",

"first_pos": "", "frequence": "2464.0Mhz", "id": "3991062614",

"image": "dimages/uav_spectrum.png",

"is_false_alarm": false,

"last_pos": "", "rc_pos": "", "seen_sensors": "2", "sequence": 39899, "severity": "1", "whitelisted": false

},

……

]

}

]

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"dat\" on type \"EventAggregation\". Did you

mean \"data\"?",

"locations": [

{

"line": 16, "column": 7

}

]

}

]

}

Events_by_paging 事件展开查询

Method：POST

Request example

query

{

events_by_paging

(

offset:0 limit:1

drone_type:"CYRF6936 General" drone_id:"3991062614" begin_time:"2024-03-14T00:00:00.564197Z" end_time:"2024-12-20T23:59:59.564197Z" filter_attacked:false

filter_severity:[SEVERITY_NA,SEVERITY_LOW,SEVERITY_HIGH,SEVERITY_MEDIUM]

filter_has_rc_pos:false filter_has_pos:false filter_has_trace:false order:"desc" order_by:"drone_type" is_false_alarm:false

)

{

count

data

{

attacked blacklisted created_time deleted_time drone_type first_pos frequence has_screenshot id

image is_false_alarm last_pos

rc_pos reliability seen_sensors sequence severity siteLatitude siteLongitude whitelisted

}

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| offset | Int | 从该⽂件查询 |
| limit | Int | 每⻚记录数，默认10 |
| drone_type | String | 按⽆⼈机类型筛选 |
| drone_id | String | 按⽆⼈机ID过滤 |
| is_false_alarm | Boolean | 是否查看误报 |
| begin_time | String | 查询开始时间 |
| end_time | String | 查询结束时间 |

| Name | Type | Description |
| --- | --- | --- |
| filter_attacked | Boolean | 筛选是否被⼲扰历史事件 |
| filter_severity | [Severity] | 筛选⽆⼈机危险等级历史事件SEVERITY_NA：未知等级；SEVERITY_LOW：低危险等级；SEVERITY_MEDIUM：中危险等级；SEVERITY_HIGH：⾼危险等级。 |
| filter_has_rc_pos | Boolean | 筛选是否有⻜⼿位置历史事件 |
| filter_has_pos | Boolean | 筛选是否有⽆⼈机位置历史事件 |
| filter_has_trace | Boolean | 筛选是否有轨迹历史事件 |
| order | String | 数据顺序，asc升序,desc降序，默认降序 |
| order_by | String | 按此⽂件排序，值：created_time：检测时间，duration：⽆⼈机持续时间，drone_id：⽆⼈机 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| count | Int | 记录数 |
| data | String | ⽆⼈机历史事件 |
| attacked | Boolean | 是否受到攻击 |
| blacklisted | Boolean | 是否加⼊⿊名单 |
| created_time | DateTime | 创建时间 |
| deleted_time | DateTime | 删除时间 |
| drone_type | String | ⽆⼈机类型 |
| first_pos | String | ⾸次发现位置 |
| frequence | String | 频率 |
| has_screenshot | Boolean | 是否有截图 |
| id | String | ⽆⼈机唯一ID |
| image | String | ⽆⼈机图像 |
| is_false_alarm | Boolean | 是否标记为误报 |
| last_pos | String | 最后发现位置 |
| rc_pos | String | ⽆⼈机⻜⼿位置 |
| reliability | Int | 该历史事件可信度，0 表⽰⽆效 |

| Name | Type | Description |
| --- | --- | --- |
| seen_sensors | String | 由哪个传感器看到的 |
| sequence | String | 事件序列号 |
| severity | Int | 严重程度：0：不可⽤（未知），1：低 2：中 3：⾼ |
| siteLatitude | Float | 站点纬度，210--表⽰⽆效 |
| siteLongitude | Float | 站点经度，210--表⽰⽆效 |
| whitelisted | Boolean | 是否加⼊⽩名单 |

Response Examples

Success

{

"data": {

"events_by_paging": {

"count": 4, "data": [

{

"attacked": false, "blacklisted": true,

"created_time": "2024-10-17T07:04:57.223576Z", "deleted_time": "2024-10-17T07:05:07.184225Z", "drone_type": "CYRF6936 General",

"first_pos": "", "frequence": "2464.0Mhz", "has_screenshot": false, "id": "3991062614",

"image": "dimages/uav_spectrum.png",

"is_false_alarm": false, "last_pos": "",

"rc_pos": "", "seen_sensors": "2", "sequence": 39901, "severity": "1", "whitelisted": false

}

]

}

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"offse\" on field \"events_by_paging\" of type

\"Query\". Did you mean \"offset\"?",

"locations": [

{

"line": 5,

"column": 5

}

]

}

]

}

GetSigParseImage 获取信号分析图⽚

Method：POST

Request example

query

{

getSigParseImage

(

sensorId:"1"

)

}

Parameter Description

| Name | Type | Description |
| --- | --- | --- |
| sensorId | String | 传感器 ID |

Response Examples

Success

{

"data": {

"getSigParseImage": [ "2380.jpg", "2442.jpg",

……

"5925.jpg"

]

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"sensorI\" on field \"getSigParseImage\" of type

\"Query\". Did you mean \"sensorId\"?",

"locations": [

{

"line": 5, "column": 3

}

]

}

]

}

Get_gvs_conf 获取光电设备配置

Method：POST

Request example

query

{

get_gvs_conf

(

id:"1"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | 设备ID |

Response Examples

Success

{

"data": {

"get_gvs_conf": "LogMode: info\nSaveNarrowObjsMode: 1\nPeriodicSaveBg: true\nPeriodicSaveBgInterval: 30\nSavePlanesMode: 2\nSaveBirdsMode: 1\nSaveDronesMode: 2\nDeviceId: \"1\"\nDeviceLocationLat: 30.123\nDeviceLocationLng: 116.116\nDeviceLocati…… 1500\nDetectDroneMinDistance: 50\nTorchVersion: libtorch2.0\nTrackServiceIP: 127.0.0.1\nTrackServicePort: 9999\nAutoTrackEnable: true\nRotateStopTime: 0\nPushStreamEnable: true\nWideVideoKeepAlive: false\nMaxAutoTrackTime: 30\nTrackVideoSaveRate: 1\nTrackMainTargetColor: Red\nTrackOthreTargetColor: Blue\nExcludePostList:\n- p120_t0_z6_wide1\n- p0_t0_z2_wide0\n- p0_t345_z2_wide0\n- p210_t0_z2_wide0\n- p180_t0_z2_wide0\n- p150_t0_z2_wide0\n- p120_t0_z2_wide0\n- p90_t0_z2_wide0\n- p60_t0_z2_wide0\n- p30_t0_z2_wide0\n"

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"ide\" on field \"get_gvs_conf\" of type

\"Query\". Did you mean \"id\"?",

"locations": [

{

"line": 5, "column": 4

}

]

}

]

}

GfskData 通⽤检测器列表信息

Method：POST

Request example

query

{

gfskData

{

bandwidth bitrate chan_lis created_time options protocol

uid

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| bandwidth | Int | GFSK带宽（对应⾼级_ 通⽤检测_ 带宽） |
| bitrate | Int | GFSK的⽐特率（对应⾼级_ 通⽤检测_ ⽐特率 单位bps） |
| chan_lis | String | 对应⾼级_ 通⽤检测_ 信道 |
| created_time | DateTime | GFSK 的创建时间 |
| options | String | 指定 GFSK 协议类型的条件（对应⾼级_ 通⽤检测_ 协议类型） |
| protocol | String | GFSK 协议 |
| uid | Int | GFSK 类型的唯一 ID |

Response Examples

Success

{

"data": {

"gfskData": [

{

"bandwidth": 250000, "bitrate": 80000, "chan_lis":

"2428599100,2433397900,2434597600,2435797300,2436997000,2438196700,2441795800,2444195200,244

5394900,2447794300,2448994000,2450193700,2457391900,2458591600,2460991000",

"created_time": "2022-05-06T06:43:39Z", "options": "

{\"preamble_field_len\":0,\"sync_field\":\"0xFDFEFDFE\",\"length_field_check\":true,\"payloa

d_field_len\":34,\"dewhitening\":false,\"crc\":false,\"fec\":false}",

"protocol": "CC2500", "uid": 13

},

……

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"ui\" on type \"GFSK\". Did you mean \"uid\"?", "locations": [

{

"line": 11, "column": 4

}

]

}

]

}

GfskProto 通⽤检测器协议信息

Method：POST

Request example

{

gfskProto

{

A7106 BK5811 CC2500 CYRF6936

GFSK_Customized

options

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| A7106 | [String] | A7106协议 |
| BK5811 | [String] | BK5811协议 |
| CC2500 | [String] | CC2500协议 |

| Name | Type | Description |
| --- | --- | --- |
| CYRF6936 | [String] | CYRF6936协议 |
| GFSK_Customized | [String] | GFSK_Customized协议 |
| option | [String] | 协议选项 |

Response Examples

Success

{

"data": {

"gfskProto": {

"A7106": [ "payload_field_len", "fec",

"crc" ], "BK5811": [

"payload_field_len"

],

"CC2500": [ "preamble_field_len", "sync_field", "length_field_check", "payload_field_len", "dewhitening",

"fec",

"crc"

],

"CYRF6936": [ "preamble_field_len", "sop_len"

],

"GFSK_Customized": [ "packet_format", "mod_scheme", "preamble_field", "sync_field_idx", "sync_field", "length_field_idx", "length_field_len", "payload_field_idx", "payload_field_len", "fec",

"crc", "crc_reflect_data", "crc_reflect_remainder", "crc_init_remainder", "crc_final_xor_val"

],

"options": [ "A7106", "CC2500", "BK5811", "CYRF6936",

"GFSK_Customized"

]

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"option\" on type \"ProtolcolType\". Did you

mean \"options\"?",

"locations": [

{

"line": 9, "column": 5

}

]

}

]

}

LinkNodeList 组⽹节点（已移⼊列表）

Method：POST

Request example

query

{

linkNodeList

{

id info ip

latitude longitude name

type

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| id | String | 节点ID |
| info | String | 节点描述 |
| ip | String | 节点IP地址 |
| latitude | Float | 节点纬度 |
| longitude | Float | 节点经度 |
| name | String | 节点名称 |
| type | String | 节点类型 |

Response Examples

Success

{

"data": {

"linkNodeList": [

{

"id": "LZA9b080443-3e81-46f9-831f-433984f7a1d0",

"info": "192.168.100.102 |", "ip": "192.168.100.102", "latitude": 0,

"longitude": 0, "name": "102", "type": "LZA"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"typ\" on type \"Node\". Did you mean

\"type\"?",

"locations": [

{

"line": 11, "column": 9

}

]

}

]

}

LocalPatchInfo 可⽤于切换的本地补丁

Method：POST

Request example

query

{

localPatchInfo (class:"engine") {

local_versions { version_id version_publish_time version_status

}

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| class | String | 补丁类型，当前仅⽀持：引擎、⽤⼾界⾯ |

Responses

| Name | Type | Description |
| --- | --- | --- |
| local_versions | String | 有效的本地版本 |
| version_id | Float | 版本的唯一 ID |
| version_publish_time | Float | 发布时间 |
| version_status | String | 版本状态和有效性 |

Response Examples

Success

{

"data": {

"localPatchInfo": {

"local_versions": [

{

"version_id": "2024.10.15", "version_publish_time": "2024-10-15T00:54:54", "version_status": "current_version"

}

]

}

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"version_statu\" on type \"versions\". Did you

mean \"version_status\", \"version_id\", or \"version_publish_time\"?",

"locations": [

{

"line": 9,

"column": 7

}

]

}

]

}

Nodes 组⽹节点信息

Method：POST

Request example

query

{

nodes

(

id:""

)

{

id info ip

latitude longitude name

type

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | 节点ID（未填或不传显⽰所有） |

Responses

| Name | Type | Description |
| --- | --- | --- |
| info | String | 节点描述 |
| ip | String | 节点IP地址 |
| id | String | 节点ID |
| latitude | Float | 节点纬度 |
| longitude | Float | 节点经度 |
| name | String | 节点名称 |
| type | String | 节点类型 |

Response Examples

Success

{

"data": {

"nodes": [

{

"id": "LZA9b080443-3e81-46f9-831f-433984f7a1d0", "info": "192.168.100.102 |",

"ip": "192.168.100.102", "latitude": 0, "longitude": 0,

"name": "102",

"type": "LZA" },

{

"id": "LZAb8bcd62b-a95f-4b14-8383-d2a94d867d2f",

"info": "192.168.100.103 |", "ip": "192.168.100.103", "latitude": 0,

"longitude": 0, "name": "103", "type": "LZA"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"typ\" on type \"Node\". Did you mean

\"type\"?",

"locations": [

{

"line": 14, "column": 7

}

]

}

]

}

Notify 通知信息

Method：POST

Request example

query{

notify(

key:13

)

{

regularNotify {

extra id info key

source status time

}

reservedNotify {

extra id info key

source status time

}

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| key | Int | 定期通知起始键 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| regularNotify | [notifyType] | 只出现一次的通知 |
| extra | String | 额外信息 |
| id | String | 设备ID（如控制器_0） |
| info | String | 信息 |
| key | Int | 节点名称 |
| source | String | 当前通知来源 |
| status | String | 通知状态 |
| time | String | 时间 |
| reservedNotify | [notifyType] | 通知总是出现 |
| extra | String | 额外信息 |
| id | String | 设备ID（如控制器_0） |
| info | String | 信息 |
| key | Int | 节点名称 |
| source | String | 当前通知来源 |
| status | String | 通知状态 |
| time | String | 时间 |

Response Examples

Success

{

"data": {

"notify": {

"regularNotify": [

{

"extra": "", "id": "2", "info": "", "key": 15,

"source": "sensor",

"status": "start",

"time": "2024-10-17 15:25:54"

},

{

"extra": "", "id": "1", "info": "", "key": 14,

"source": "jammer", "status": "start",

"time": "2024-10-17 15:25:16"

}

],

"reservedNotify": []

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"tim\" on type \"notifyType\". Did you mean

\"time\"?",

"locations": [

{

"line": 22, "column": 7

}

]

}

]

}

ProfileAliasList 查询⽆⼈机⾃定义机型列表

Method：POST

Request example

query{

profileAliasList {

alias description drone_type visible

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| alias | String | ⾃定义别名 |
| description | String | ⽆⼈机⾃定义机型备注 |
| drone_type | String | ⽆⼈机机型 |
| visible | Boolean | 显⽰⽆⼈机时为 "true"，隐藏时为 "false" |

Response Examples

Success

{

"data": {

"profileAliasList": [

{

"alias": "12378", "description": "456", "drone_type": "123", "visible": false

}

]

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"visibled\" on type \"ProfileAlias\". Did you mean

\"visible\"?",

"locations": [

{

"line": 6, "column": 5

}

]

}

]

}

ProfileList 查询⽆⼈机属性

Method：POST

Request example

query

{

profileList { description drone_id drone_link_id drone_type visible

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| description | String | ⽆⼈机说明 |
| drone_id | String | ⽆⼈机ID |
| drone_link_id | String | 与其他⽆⼈机一起减少ID |
| drone_type | String | 定制型⽆⼈机 |
| visible | Boolean | 显⽰⽆⼈机时为 "true"，隐藏时为 "false" |

Response Examples

Success

{

"data": {

"profileList": [

{

"description": "",

"drone_id": "1WNBH5B00200L5", "drone_link_id": "60601f0916ae", "drone_type": "DJI MavicAir2", "visible": true

}

]

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"visibl\" on type \"Profile\". Did you mean

\"visible\"?",

"locations": [

{

"line": 8,

"column": 4

}

]

}

]

}

ProfilePatternList 查询⽤于匹配⽆⼈机 ID 的模式列表

Method：POST

Request example

query{

profilePatternList { description drone_type

pattern

visible

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| description | String | ⽆⼈机备注 |
| drone_type | String | ⽆⼈机机型 |
| pattern | String | 正则表达式模式 |
| visible | Boolean | 显⽰⽆⼈机时为 "true"，隐藏时为 "false" |

Response Examples

Success

{

"data": {

"profilePatternList": [

{

"description": "09876", "drone_type": "67890", "pattern": "12345", "visible": true

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"drone_typed\" on type \"ProfilePattern\". Did you

mean \"drone_type\"?",

"locations": [

{

"line": 4, "column": 5

}

]

}

]

}

Report 统计Method：POST Request example

query{

report

(

from:"2025-06-10T02:55:37.665168Z" to:"2025-06-10T03:27:43.597138Z"

)

{

avg_duration avg_per_day avgdrn_per_day daily_data hourly_data longest_duration moments {

events { attacked blacklisted created_time deleted_time drone_type first_pos frequence

has_screenshot

id

image is_false_alarm last_pos

rc_pos reliability seen_sensors sequence severity siteLatitude siteLongitude whitelisted

} lost_time seen_time

} most_in_alarm_region_per_day most_per_day

most_per_hour mostdrn_in_alarm_region_per_day mostdrn_per_day

mostdrn_per_hour top_drone_type top_drones total_drones total_duration total_in_alarm_region total_incidents total_mitigation

totaldrn_in_alarm_region

totaldrn_mitigation

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| from | DateTime | 开始时间 |
| to | DateTime | 结束时间 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| avg_duration | Int | 平均持续时间 |
| avg_per_day | Int | 平均每⽇架次 |
| avgdrn_per_day | Int | 平均每⽇架数 |
| daily_data | datatype | 事件趋势_⽇趋势 |
| hourly_data | datatype | 事件趋势_⼩时趋势 |
| longest_duration | Int | 单次最⻓时间 |
| moments | momentType | 关键时刻 |
| events | [Event] | 时间范围内的事件 |
| attacked | Boolean | 是否受到攻击 |
| blacklisted | Boolean | 是否被列⼊⿊名单 |
| created_time | DateTime | 创建时间 |
| deleted_time | DateTime | 删除时间 |
| drone_type | String | ⽆⼈机类型 |
| first_pos | String | ⾸次发现位置 |
| frequence | String | 频率 |
| has_screenshot | Boolean | 是否有截图 |
| id | String | ⽆⼈机唯一ID |
| image | String | ⽆⼈机图像 |
| is_false_alarm | Boolean | 是否标记为误报 |
| last_pos | String | 最后发现位置 |

| Name | Type | Description |
| --- | --- | --- |
| rc_pos | String | ⽆⼈机⻜⼿位置 |
| reliability | Int | 该检测记录的可靠性，0 表⽰⽆效 |
| seen_sensors | String | 由哪个传感器看到的 |
| sequence | String | 事件序列号 |
| severity | Int | 严重程度：0：不可⽤（未知），1：低 2：中 3：⾼ |
| siteLatitude | Float | 站点的纬度，210 "表⽰⽆效 |
| siteLongitude | Float | 站点的经度，210 "表⽰⽆效 |
| whitelisted | Boolean | 是否加⼊⽩名单 |
| lost_time | DateTime | 丢失时间戳 |
| seen_time | DateTime | 发现时间戳 |
| most_in_alarm_region_per_day | Int | 预警区单⽇最多架次 |
| most_per_day | Int | 单⽇最多架次 |
| most_per_hour | Int | 单⼩时最多架次 |
| mostdrn_in_alarm_region_per_day | Int | 预警区单⽇总架数 |
| mostdrn_per_day | Int | 单⽇最多架数 |
| mostdrn_per_hour | Int | 单⼩时最多架数 |
| top_drone_type | datatype | 检测到⽆⼈机类型 |
| top_drones | datatype | 检测到⽆⼈机 |
| total_drones | Int | 总发现架数 |
| total_duration | Int | 被⼊侵总时⻓（单位 秒 换算为分） |
| total_in_alarm_region | Int | 预警区总发现架次 |
| total_incidents | Int | 总发现架次 |
| total_mitigation | Int | 总拦截架次 |
| totaldrn_in_alarm_region | Int | 预警区总发现架 |
| totaldrn_mitigation | Int | 总拦截架数 |

Response Examples

Success

{

"data": {

"report": { "avg_duration": 154, "avg_per_day": 9, "avgdrn_per_day": 9, "daily_data": [

[

"Time",

"Number of Incidents",

"Total duration of the day", "Number of Drones"

],

[

"2025-06-10T08:00:00+08:00",

9, 1394, 9

]

],

"hourly_data": [

[

"Time",

"Number of Incidents",

"Total duration of the hour",

"Number of Drones" ],

[

"2025-06-10T10:00:00+08:00",

1,

20, 1

],

[

"2025-06-10T11:00:00+08:00",

8, 1374, 8

]

],

"longest_duration": 916, "moments": [

{

"events": [

{

"attacked": false,

"blacklisted": false,

"created_time": "2025-06-10T03:02:01.59499Z", "deleted_time": "2025-06-10T03:17:17.598394Z", "drone_type": "HX/Skydroid Herelink/H16", "first_pos": "",

"frequence": "", "has_screenshot": false, "id": "d1be",

"image": "",

"is_false_alarm": false,

"last_pos": "", "rc_pos": "", "reliability": 0, "seen_sensors": "", "sequence": 11772, "severity": "0", "siteLatitude": 0, "siteLongitude": 0, "whitelisted": false

}

],

"lost_time": "2025-06-10T03:17:17.598394Z", "seen_time": "2025-06-10T03:02:01.59499Z"

}

],

"most_in_alarm_region_per_day": 0, "most_per_day": 9,

"most_per_hour": 8, "mostdrn_in_alarm_region_per_day": 0, "mostdrn_per_day": 9, "mostdrn_per_hour": 8, "top_drone_type": [

[

"Drone Type",

"Number of Incidents"

],

……

[

"FRSKY R9M_LORA",

1

]

],

"top_drones": [

[

"Drone Type", "ID",

"Number of Incidents" ],

……

[

"ELRS LoraSF6", "06b9ed",

1

]

],

"total_drones": 9, "total_duration": 1394, "total_in_alarm_region": 0, "total_incidents": 9, "total_mitigation": 0, "totaldrn_in_alarm_region": 0, "totaldrn_mitigation": 0

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"avg_durations\" on type \"reportType\". Did you mean

\"total_duration\", \"avg_duration\", or \"longest_duration\"?",

"locations": [

{

"line": 8, "column": 5

}

]

}

]

}

Screenshot 图传截图

Method：POST

Request example

{

screenshot

(

offset:1 order:"desc" seq:949 limit:1

)

{

count

data { create_time data drone_id drone_type

event_sequence

file_path

id

}

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| offset | Int | 要返回的 firest 记录的偏移量 |
| order | String | 查询顺序，从降序或升序中选择一个 |
| seq | Int | 截图事件序列号 |
| limit | Int | 返回记录的最⼤值 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| count | Int | 总记录数量 |
| data | List | 截图数据列表 |
| create_time | DateTime | 创建时间 |
| data | String | 截图数据 |
| drone_id | String | ⽆⼈机ID |
| drone_type | String | ⽆⼈机类型 |
| event_sequence | String | 事件序列 |
| file_path | String | ⽂件路径 |
| id | String | 截图ID |

Response Examples

Success

{

"data": {

"screenshot": { "count": 397, "data": [

{

"create_time": "2024-10-11T22:28:03.99+08:00", "data":

"/9j/4AAQSkZJRgABAQAAAAAAAAD/2wBDAAMCAgMCAgMDAwMEAwMEBQgFBQQEBQoHBwYIDAoMDAsKCwsNDhIQDQ4RDgs LEBYQERMUFRUVDA8XGBYUGBIUFRT/2wBDAQMEBAUEBQkFBQkUDQsNFBQUFBQUFBQUFBQUFBQUFBQUFBQUFBQUFBQUFBQ UFBQUFBQUFBQUFBQUFBQUFBQUFBT/wAARCAIQAtADASIAAhE…… 9KsmFRkGHI/wB/1qGRR/z65PX/AFlAFSaKMcOM9u/4VWcWgz8v6nvV5ojyZcDp2q3G7gdff/GgBEGRjfnt0qVc5/1u0f 7vrQGkP8u1Tx5zkjj/ABoAfEcn7+8/TFWUjHTZ+v5U2Pb/ABRZ/wCBVet44DgfZuen+sNAEMKxlhmLPtuq47W9nCXe26 DrvNRX2p2WlwFmg5xnG9v8K8U+IXjiO7aSKG1yfXzD",

"drone_id": "80ff2b", "drone_type": "DIY FPV",

"event_sequence": 949,

"file_path": "/opt/lz/screenshot/2024-10-11/80ff2b/80ff2b-14-28-03.990.img", "id": 4

}

]

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"valu\" on type \"request\". Did you mean

\"value\"?",

"locations": [

{

"line": 7, "column": 4

}

]

}

]

}

Sensor 传感器Method：POST Request example

query

{

sensor

(

id:"1"

)

{

config faults id

mac name node

sensor_status { built_time component_type first_seen git_hash ip_address

last_seen temperature version

} state ttl

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | Int | 传感器ID（不传显⽰所有） |

Responses

| Name | Type | Description |
| --- | --- | --- |
| config | JsonStrType | 传感器配置 |
| faults | String | 传感器故障信息 |
| id | String | 传感器的唯一 ID |
| mac | String | 传感器的 Mac 地址 |
| name | String | 传感器名称 |
| node | String | 传感器所属的节点 |
| sensor_status | State | 当前传感器的状态 |
| built_time | String | SCV/控制器/传感器的唯一构建时间 |
| component_type | String | SCV/控制器/传感器的组件类型 |
| first_seen | String | 当前 SCV/控制器/传感器的登录时间 |
| git_hash | String | scv/controller/sensor 的唯一 git 散列编号 |
| ip_address | String | scv/ 控制器/传感器的唯一 IP 地址 |
| last_seen | String | 当前 SCV/控制器/传感器的注销时间 |
| temperature | Int | scv/ 控制器/传感器的温度 |
| version | String | scv/ 控制器/传感器的唯一版本号 |
| state | String | 传感器的当前状态，处于 “上升 ”或 “下降 ”状态 |
| ttl | Int | 存活时间（传感器过期时间） |

Response Examples

Success

{

"data": {

"sensor": [

{

"config": " {\"version\":\"\",\"build_time\":\"\",\"git_hash\":\"\",\"device_type\":\"DetectionOnly\",\" mqtts_ip\":\"192.178.1.88\",\"mqtts_hostname\":\"1.dcn.local\",\"id\":\"1\",\"group\":\"At\" ,\"mac\":\"06:00:20:0A:8C:04\",\"sn\":\"S111111117540001P901\",\"pid\":\"DAFT- W1590DPI\",\"hardware_version\":\"2\",\"rfchain_master\":true,\"ttl\":15,\"geo_location\": {\"lat\":40.0423,\"lng\":116.4736},\"location_locked\":false,\"range\":8000,\"mimo_num\":1,\ "eng_port\":\"8000\",\"eng_ip\":\"192.178.1.88\",\"n_eng\":2,\"listen_wifi\":\"\",\"watchdog _enable\":false,\"compass_enable\":true,\"dfant_id\":\"\",\"crif_califile\":\"crif_cali.bin\ ",\"is_sdexist\":true,\"cps_offset\":0,\"bands\": [\"band24\",\"band58\",\"band14\",\"band9\",\"band4\"],\"with_spf\":false}",

"faults": "",

"id": "1",

"mac": "06:00:20:0A:8C:04", "name": "",

"node": "master",

"sensor_status": { "built_time": "", "component_type": null,

"first_seen": "18 Oct 24 09:19 CST",

"git_hash": "",

"ip_address": "192.178.1.251/24", "last_seen": "",

"temperature": null, "version": ""

},

"state": "operational",

"ttl": 12

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"tt\" on type \"Sensor\". Did you mean

\"ttl\"?",

"locations": [

{

"line": 25, "column": 5

}

]

}

]

}

SpdReport 频谱

Method：POST

Request example

query

{

spdReport

(

freq_start: 2400 freq_stop: 2500

)

{

bandwidth description frequence

}

}

Parameter Description

| Name | Type | Description |
| --- | --- | --- |
| freq_start | Int | 频谱跨度的起始频率，单位为Khz |
| freq_stop | String | 频谱扫描的终⽌频率，单位Khz |
| bandwidth | Int | 信号带宽 |
| description | Int | 信号描述 |
| frequence | Int | 信号频率 |

Response Examples

Success

{

"data": {

"spdReport": [

{

"bandwidth": 13980, "description": "FM调频", "frequence": 101400

},

……

{

"bandwidth": 1740, "description": "", "frequence": 116220

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"frequenc\" on type \"spdReport\". Did you mean

\"frequence\"?",

"locations": [

{

"line": 11, "column": 4

}

]

}

]

}

SpoofStatus 诱骗器状态

Method：POST

Request example

query

{

spoofStatus {

switch

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| switch | Int | 开关状态，0:关闭，1:打开，2:迫降，3:驱离。 |

Response Examples

Success

{

"data": {

"spoofStatus": {

"switch": 0

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"switc\" on type \"spoofStatus\". Did you mean

\"switch\"?",

"locations": [

{

"line": 4, "column": 4

}

]

}

]

}

Storage 设备存储空间

Method：POST

Request example

query

{

storage { externalSize internalSize

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| externalSize | Float | 设备外部可⽤空间（以 MB 为单位） |
| internalSize | Float | 设备内部可⽤空间（以 MB 为单位） |

Response Examples

Success

{

"data": {

"storage": {

"externalSize": 80961.80859375,

"internalSize": 0

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"internalSiz\" on type \"storage\". Did you mean

\"internalSize\" or \"externalSize\"?",

"locations": [

{

"line": 5, "column": 4

}

]

}

]

}

SysCapability 设备⽀持打击⽅式

Method：POST

Request example

query

{

sysCapability

}

Parameter Description

None

Response Examples

Success

{

"data": {

"sysCapability": [ "Wideband", "Smart", "Smart-II"

]

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"sysCapabilit\" on type \"Query\". Did you mean

\"sysCapability\"?",

"locations": [

{

"line": 3,

"column": 2

}

]

}

]

}

Time 查看设备系统时间

Method：POST

Request example

query

{

time {

sysTime

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| sysTime | String | 系统时间 |

Response Examples

Success

{

"data": {

"time": {

"sysTime": "2024-03-26 13:56"

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"sysTim\" on type \"time\". Did you mean

\"sysTime\"?",

"locations": [

{

"line": 4, "column": 4

}

]

}

]

}

TimeZone 查看设备时区

Method：POST

Request example

query

{

timeZone { allTimeZone sysTimeZone

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| allTimeZone | String | 所有时区 |
| sysTimeZone | String | 系统时区 |

Response Examples

Success

{

"data": {

"timeZone": {

"allTimeZone": [ "Africa/Abidjan", "Africa/Accra", "Africa/Addis_Ababa", "Africa/Algiers", "Africa/Asmara",

…… "Universal", "W-SU", "WET",

"Zulu"

],

"sysTimeZone": "Asia/Shanghai"

}

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"sysTimeZon\" on type \"timeZone\". Did you mean

\"sysTimeZone\" or \"allTimeZone\"?",

"locations": [

{

"line": 5,

"column": 7

}

]

}

]

}

UnknownSignal 未知信号

Method：POST

Request example

query

{

unknownSignal

{

bw freq mod

seen_sensors

time

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| bw | String | 信号频宽 |
| freq | String | 信号频率 |
| mod | String | 信号调制⽅案 |
| seen_sensors | String | 检测到信号的传感器 |
| time | String | 信号时间 |

Response Examples

Success

{

"data": {

"unknownSignal": []

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"tim\" on type \"unknownSignal\". Did you mean

\"time\"?",

"locations": [

{

"line": 9,

"column": 4

}

]

}

]

}

UnlinkNodeList 组⽹未知节点列表

Method：POST

Request example

query

{

unknownSignal

{

bw freq mod

seen_sensors

time

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| conn_type | String | 连接类型 |
| id | String | 站点ID |
| info | String | 站点信息 |
| name | String | 站点名称 |

Response Examples

Success

{

"data": {

"unlinkNodeList": [

{

"conn_type": "register",

"id": "LZAf7a94b8c-1438-4178-b20a-601c9ab73486",

"info": "", "name": "940"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"nam\" on type \"site\". Did you mean

\"name\"?",

"locations": [

{

"line": 8, "column": 5

}

]

}

]

}

Upgrade 本地升级版本

Method：POST

Request example

query

{

upgrade { free_space has_enough_space local_versions {

version_id version_publish_time version_status

} prepare_status upgrade_status

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| free_space | Int | 新升级的可⽤空间（以 MB 为单位） |
| has_enough_space | Boolean | 是否有⾜够的空间进⾏新的升级 |
| local_versions | String | 有效的本地版本 |
| version_id | String | 版本唯一ID |
| version_publish_time | String | 版本发布时间 |

| Name | Type | Description |
| --- | --- | --- |
| version_status | String | 版本状态和有效性 |
| prepare_status | String | 准备升级⽬录 |
| upgrade_status | String | 升级状态 |

Response Examples

Success

{

"data": {

"upgrade": {

"free_space": 0, "has_enough_space": false, "local_versions": [

{

"version_id": "v2024.09.09", "version_publish_time": "2024-09-09T01:03:01Z", "version_status": "Valid"

},

……, {

"version_id": "v3.14.0",

"version_publish_time": "2024-10-12T10:21:05Z",

"version_status": "Valid"

}

],

"prepare_status": "", "upgrade_status": "updated"

}

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Cannot query field \"upgrade_statu\" on type \"upgradeType\". Did

you mean \"upgrade_status\" or \"prepare_status\"?",

"locations": [

{

"line": 12,

"column": 5

}

]

}

]

}

Version 当前版本信息

Method：POST

Request example

query

{

version

}

Parameter Description

None

Response Examples

Success

{

"data": {

"version": "controller\n\tversion: 2024.10.15, built at 2024-10-15T01:01:25Z, git hash:f448b89e4528165bf16c6a91ae1d792d4c45f8b5\n\ncm\n\tversion: 2024.10.15, built at 2024- 10-15T01:01:51Z, git hash:7c0d0cf78f71ff2a5a193f0e6d1695381b909df5\n\nengine/'4418'\n\tversion: 2024.10.15, built at 2024-10-15T00:54:54, …… \n/opt/lz:\t\t[0 - 100%]\n<log>: 918M <sigrecords>: 4.0K

<sigdump>: 4.0K  <sigcollect>: 4.0K <sigrecordcrash>: 4.0K <screenshot>: 96M <upload>:

4.0K \n"

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"versio\" on type \"Query\". Did you mean

\"version\"?",

"locations": [

{

"line": 3, "column": 2

}

]

}

]

}

Vsgevent 光电单事件查询

Method：POST

Request example

query

{

vsgevent

(

uuid:"eeb8ce4b-9633-4fdd-8ba3-5769057bbbc2"

)

{

distance_between_obj_and_cam

drone_type firstseen_ptz firstseen_size firstseen_time icon

id

image lastseen_ptz lastseen_size lastseen_time seen_sensors video

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| uuid | String | ⽆⼈机ID |

Responses

| Name | Type | Description |
| --- | --- | --- |
| distance_between_obj_and_cam | Int | 估计距离 |
| drone_type | String | 预测的⽆⼈机类型 |
| firstseen_ptz | String | 第一次看到云台 |
| firstseen_size | String | ⾸次看到的尺⼨ |
| firstseen_time | DateTime | 初⻅时间 |
| icon | String | 图标⽂件路径 |

| Name | Type | Description |
| --- | --- | --- |
| id | String | ⽆⼈机的 UUID |
| image | String | 原始检测图像⽂件名 |
| lastseen_ptz | String | 最后看到的 PTZ |
| lastseen_size | String | 最后看到的尺⼨ |
| lastseen_time | DateTime | 最后出现时间 |
| seen_sensors | String | 由哪个传感器发现 |
| video | String | 跟踪视频⽂件名 |

Response Examples

Success

{

"data": {

"vsgevent": [

{

"distance_between_obj_and_cam": 17, "drone_type": "mavic", "firstseen_ptz": "p59.3t350.5z2.0", "firstseen_size": "143x51",

"firstseen_time": "2024-10-22T06:28:31+08:00", "icon": "dimages/mavic_pro.png",

"id": "eeb8ce4b-9633-4fdd-8ba3-5769057bbbc2", "image":

"/9j/4AAQSkZJRgABAQAAAQABAAD/2wBDAAIBAQEBAQIBAQECAgICAgQDAgICAgUEBAMEBgUGBgYFBgYGBwkIBgcJBwY GCAsICQoKCgoKBggLDAsKDAkKCgr/2wBDAQICAgICAgUDAwUKBwYHCgoKCgoKCgoKCgoKCgoKCgoKCgoKCgoKCgoKCgo KCgoKCA…… +vA59qcdyDjNQ8MaT4yF/qHhsrMDpUbO0sslvYRJAC00skjhjHg+WmAuGlPXnjzjXfBtroK3+v2Wl6Zb6jb6vJdXt7LZ xXNnd6e+PMtEdiXduHqfimKO+8SeHoL2NZkudZCXCSjcJVCggMD94A8gHvXL63ZWd74vbS7y0iltk1VQlvLGGjUNJ8wC ngZ7+tWJ7H/2Q==",

"lastseen_ptz": "p55.0t336.5z2.0",

"lastseen_size": "136x44",

"lastseen_time": "2024-10-22T09:21:16+08:00", "seen_sensors": "1",

"video": "20241022_09-20-17.7921.mp4"

}

]

}

}

Failure

{

"data": {

"vsgevent": []

},

"errors": [

{

"message": "sql: no rows in result set", "locations": [

{

"line": 3, "column": 2

}

],

"path": [

"vsgevent"

]

}

]

}

Vsgevents 光电多事件查询

Method：POST

Request example

query

{

vsgevents

(

from:1 to:2

)

{

distance_between_obj_and_cam

drone_type firstseen_ptz firstseen_size firstseen_time icon

id

image lastseen_ptz lastseen_size lastseen_time seen_sensors video

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| from | Int | 从开始 |
| to | Int | 结束 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| distance_between_obj_and_cam | Int | 估计距离 |
| drone_type | String | 预测的⽆⼈机类型 |
| firstseen_ptz | String | 第一次看到云台 |
| firstseen_size | String | ⾸次看到的尺⼨ |
| firstseen_time | DateTime | 初⻅时间 |
| icon | String | 图标⽂件路径 |
| id | String | ⽆⼈机的 UUID |
| image | String | 原始检测图像⽂件名 |
| lastseen_ptz | String | 最后看到的 PTZ（P：⽅向⻆；t：俯仰⻆；z：倍率） |
| lastseen_size | String | 最后看到的尺⼨ |
| lastseen_time | DateTime | 最后出现时间 |
| seen_sensors | String | 由哪个传感器发现 |
| video | String | 跟踪视频⽂件名 |

Response Examples

Success

{

"data": {

"vsgevents": [

{

"distance_between_obj_and_cam": 7, "drone_type": "mavic", "firstseen_ptz": "p25.7t348.8z2.0", "firstseen_size": "278x139",

"firstseen_time": "2024-03-26T17:13:06+08:00",

"icon": "dimages/mavic_pro.png",

"id": "a96bb6c5-bcab-4547-8adf-3c2c4a7b7760", "image": "",

"lastseen_ptz": "p25.7t348.8z2.0", "lastseen_size": "278x139",

"lastseen_time": "2024-03-26T17:14:13+08:00",

"seen_sensors": "1",

"video": ""

},

……,

{

"distance_between_obj_and_cam": 30, "drone_type": "mavic", "firstseen_ptz": "p131.0t1.9z6.0", "firstseen_size": "177x73",

"firstseen_time": "2024-05-11T17:50:31+08:00", "icon": "dimages/mavic_pro.png",

"id": "db373e96-671d-4784-b979-1d5a6fde5bef",

"image": "",

"lastseen_ptz": "p141.7t0.2z6.0", "lastseen_size": "201x105",

"lastseen_time": "2024-05-11T17:52:07+08:00",

"seen_sensors": "1", "video": ""

}

]

}

}

Failure

{

"data": {

"vsgevent": []

},

"errors": [

{

"message": "sql: no rows in result set",

"locations": [

{

"line": 3, "column": 2

}

],

"path": [

"vsgevent"

]

}

]

}

Vsgeventsimage 光电图⽚查询

Method：POST

Request example

query

{

vsgeventsimage

(

uuid:"eeb8ce4b-9633-4fdd-8ba3-5769057bbbc2"

history:false

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| uuid | Int | 从开始 |
| history | Int | true：查询历史事件，false：当前⽆⼈机列表 |

Response Examples

Success

{

"data": {

"vsgeventsimage": "/9j/4AAQSkZJRgABAQAAAQABAAD/2wBDAAIBAQEBAQIBAQECAgICAgQDAgICAgUEBAMEBgUGBgYFBgYGBwkIBgcJBwY GCAsICQoKCgoKBg…… yyzrFsAAVFLYXduHqfimKO+8SeHoL2NZkudZCXCSjcJVCggMD94A8gHvXL63ZWd74vbS7y0iltk1VQlvLGGjUNJ8wCng Z7+tWJ7H/2Q=="

}

}

Failure

{

"data": null, "errors": [

{

"message": "Argument \"history\" has invalid value fals.\nExpected type

\"Boolean\", found fals.",

"locations": [

{

"line": 6, "column": 13

}

]

}

]

}

Whitelist ⽩名单列表

Method：POST

Request example

query

{

whitelist { alias description dronetype

id

}

}

Parameter Description

Responses

| Name | Type | Description |
| --- | --- | --- |
| alias | String | ⽆⼈机别名 |
| description | String | ⽆⼈机描述 |
| dronetype | String | ⽆⼈机类型 |
| id | String | ⽆⼈机唯一ID |

Response Examples

Success

{

"data": {

"whitelist": [

{

"alias": "DJI Mavic3",

"description": "permanent,permanent",

"dronetype": "",

"id": "F45TB22BF00E021U"

}

……

{

"alias": "",

"description": "2024-03-26 13:23:43,2024-04-26 13:23:43", "dronetype": "3DR Solo",

"id": "undefined"

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"i\" on type \"Whitelist\". Did you mean

\"id\"?",

"locations": [

{

"line": 7, "column": 5

}

]

}

]

}

WidebandJammer 宽频⼲扰状态

Method：POST

Request example

query

{

widebandJammer

(

directional:false

)

{

band12 band14 band15 band18 band24 band4 band58 band9

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| directional | Int | 定向⼲扰器 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| band12 | Int | 频段状态 1.2G，0:不可⽤，1:空闲，其他:繁忙。 |
| band14 | Int | 频段状态 1.4G，0:不可⽤，1:空闲，其他:繁忙。 |
| band15 | Int | 频段状态 1.5G，0:不可⽤，1:空闲，其他:繁忙。 |
| band18 | Int | 频段状态 1.8G，0:不可⽤，1:空闲，其他:繁忙。 |
| band24 | Int | 频段状态 2.4G，0:不可⽤，1:空闲，其他:繁忙。 |
| band4 | Int | 频段状态 433M，0:不可⽤，1:空闲，其他:繁忙。 |
| band58 | Int | 频段状态 5.8G，0:不可⽤，1:空闲，其他:繁忙。 |
| band9 | Int | 频段状态900M，0:不可⽤，1:空闲，其他:繁忙。 |

Response Examples

Success

{

"data": {

"widebandJammer": { "band12": 1, "band14": 1, "band15": 1, "band18": 1, "band24": 1, "band4": 1, "band58": 1, "band9": 1

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"band\" on type \"WidebandJammer\". Did you mean

\"band4\", \"band58\", \"band9\", \"band12\", or \"band14\"?",

"locations": [

{

"line": 15, "column": 4

}

]

}

]

}

WidebandJammer2 宽频⼲扰状态

Method：POST

Request example

query

{

widebandJammer2

(

directional:false

)

{

band12 band14 band15 band18 band24 band4 band58 band9

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| directional | Int | 定向⼲扰器 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| band12 | Int | 频段状态 1.2G，0:不可⽤，1:空闲，其他:繁忙。 |
| band14 | Int | 频段状态 1.4G，0:不可⽤，1:空闲，其他:繁忙。 |
| band15 | Int | 频段状态 1.5G，0:不可⽤，1:空闲，其他:繁忙。 |
| band18 | Int | 频段状态 1.8G，0:不可⽤，1:空闲，其他:繁忙。 |
| band24 | Int | 频段状态 2.4G，0:不可⽤，1:空闲，其他:繁忙。 |
| band4 | Int | 频段状态 433M，0:不可⽤，1:空闲，其他:繁忙。 |
| band58 | Int | 频段状态 5.8G，0:不可⽤，1:空闲，其他:繁忙。 |
| band9 | Int | 频段状态900M，0:不可⽤，1:空闲，其他:繁忙。 |

Response Examples

Success

{

"data": {

"widebandJammer": { "band12": 1, "band14": 1, "band15": 1, "band18": 1, "band24": 1, "band4": 1, "band58": 1, "band9": 1

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"band\" on type \"WidebandJammer\". Did you mean

\"band4\", \"band58\", \"band9\", \"band12\", or \"band14\"?",

"locations": [

{

"line": 15, "column": 4

}

]

}

]

}

Wifilist wifi列表

Method：POST

Request example

query

{

wifilist

(

type:"inactive"

)

{

mac reliability ssid

status

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| type | String | 选择活动/⾮活动/背景列表 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| mac | String | ⽆⼈机的 MAC 地址 |
| reliablitiy | String | ⽆⼈机的可靠性 |
| ssid | String | ⽆⼈机的 SSID |
| status | Int | ⽆⼈机的状态 |

Response Examples

Success

{

"data": {

"wifilist": [

{

"mac": "2c:16:db:a3:1e:8c",

"reliability": "0", "ssid": "", "status": 0

},

……,

{

"mac": "f0:1e:34:14:32:4e",

"reliability": "0", "ssid": "",

"status": 0

}

]

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"statu\" on type \"wifilist\". Did you mean

\"status\"?",

"locations": [

{

"line": 11, "column": 5

}

]

}

]

}

Mutation 操作

AddBlacklist 添加⿊名单

Method：POST

Request example

Requests

mutation{

addBlacklist

(

id:"7e70db030e7b123" dronetype:"DESKTOP-BLGFE4D 1341" alias:"123"

)

}

Parameter Description

| Name | Type | Description |
| --- | --- | --- |
| id | String | ⽆⼈机id |
| dronetype | String | ⽆⼈机机型 |

| Name | Type | Description |
| --- | --- | --- |
| alias | String | ⽆⼈机别名 |

Response Examples

成功

{

"data": {

"addBlacklist": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"dronetyp\" on field \"addBlacklist\" of type

\"Mutation\". Did you mean \"dronetype\"?",

"locations": [

{

"line": 5, "column": 13

}

]

}

]

}

AddWhitelist 添加⽩名单

Method：POST

Request example

mutation{

addWhitelist

(

id:"7e70db030e7b123" dronetype:"DESKTOP-BLGFE4D 1341" alias:"123" timerange:"permanent,permanent"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | ⽆⼈机id |
| dronetype | String | ⽆⼈机机型 |
| alias | String | ⽆⼈机别名 |
| timerange | String | 时效 "permanent,permanent"--永久"2024-10-18T06:23:57.000Z,2024-10-19T15:59:59.999Z"--时间格式 |

Response Examples

Success

{

"data": {

"addWhitelist": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"timerang\" on field \"addWhitelist\" of type

\"Mutation\". Did you mean \"timerange\"?",

"locations": [

{

"line": 6, "column": 13

}

]

}

]

}

AirDefenceArea 绘制防空区

Method：POST

Request example

mutation{ airDefenceArea (

data:"{\"type\":0,\"distance\":388,\"locations\":

[[116.43229,39.557729]],\"hash\":\"436968595\",\"areatype\":1}"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| data | String | 类型： 0-删除, 1-添加 |

Response Examples

Success

{

"data": {

"airDefenceArea": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"date\" on field \"airDefenceArea\" of type

\"Mutation\". Did you mean \"data\"?",

"locations": [

{

"line": 4,

"column": 9

}

]

}

]

}

Attack 精准打击

Method：POST

Request example

mutation

{

attack

(

id:"a697b160cabb"

cancel:true

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | 已检测到⽆⼈机ID |
| cancel | boolean | 如果设置，则取消对⽆⼈机的攻击 |

Response Examples

Success

{

"data": {

"attack": null

}

}

Failure

{

"data": {

"attack": null

},

"errors": [

{

"message": "Can't find drone with id 'a697b160cabb'",

"locations": [

{

"line": 3, "column": 9

}

],

"path": [

"attack"

]

}

]

}

AutoAttack ⽆⼈值守

Method：POST

Request example

mutation

{

autoAttack

(

naviBands:[band12] timeRanges:[["12:12:12","13:13:13"],["14:14:14","15:15:15"],["15:16:16","17:17:17"]] naviEnabled:false

on:false saEnabled:false wbEnabled:true dirWbEnabled:false spfEnabled:false protectWhite:false

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| on | boolean | 开关按钮 |
| saEnabled | boolean | 精准打击 |
| wbEnabled | boolean | 宽频⼲扰 |
| dirWbEnabled | boolean | 定向打击 |
| spfEnabled | boolean | GNSS诱骗 |
| protectWhite | boolean | ⽩名单保护 |
| naviBands | [naviBands] | 导航防御频段：band12/band15（1.2GHz/1.5GHz） |
| timeRanges | [[String]] | 时间范围列表，格式为[[start，end],[]..] 每个时间都是HH:MM:SS格式，代表一个每⽇时间范围；⽀持跨⽇范围。 |
| naviEnabled | Boolean | 导航防御 |

Response Examples

Success

{

"data": {

"autoAttack": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"protectWhit\" on field \"autoAttack\" of type

\"Mutation\". Did you mean \"protectWhite\"?",

"locations": [

{

"line": 10, "column": 9

}

]

}

]

}

ClearStorage 清除内部存储

Method：POST

Request example

mutation

{

clearStorage

}

Parameter Description

None

Response Examples

Success

{

"data": {

"clearStorage": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"clearStorag\" on type \"Mutation\". Did you

mean \"clearStorage\" or \"clearLogs\"?",

"locations": [

{

"line": 3, "column": 3

}

]

}

]

}

Ctrl_landing 迫降

Method：POST

Request example

mutation

{

ctrl_landing

(

id:"60601f0a5004" enable:true

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | ⽆⼈机id |
| enable | Boolean | 如果为 "是"，则开始控制⽆⼈机着陆。 |

Response Examples

Success

{

"data": {

"ctrl_landing": null

},

"errors": [

{

"message": "not support", "locations": [

{

"line": 1, "column": 10

}

],

"path": [

"ctrl_landing"

]

}

]

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"enabl\" on field \"ctrl_landing\" of type

\"Mutation\". Did you mean \"enable\"?",

"locations": [

{

"line": 6, "column": 5

}

]

}

]

}

DeleteBlacklist 移除⿊名单⽆⼈机

Method：POST

Request example

mutation

{

deleteBlacklist

(

id:"3qwe2ZXCwqeq1"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | ⽆⼈机id |

Response Examples

Success

{

"data": {

"deleteBlacklist": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"i\" on field \"deleteBlacklist\" of type

\"Mutation\". Did you mean \"id\"?",

"locations": [

{

"line": 5, "column": 3

}

]

}

]

}

DeleteEvents 删除事件

Method：POST

Request example

mutation

{

deleteEvents

(

sequence:[123]

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| sequence | [Int] | 要删除的事件序列数 |

Response Examples

Success

{

"data": {

"deleteEvents": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"sequenc\" on field \"deleteEvents\" of type

\"Mutation\". Did you mean \"sequence\"?",

"locations": [

{

"line": 5,

"column": 3

}

]

}

]

}

DeleteEventsByID 根据⽆⼈机ID删除事件

Method：POST

Request example

mutation

{

deleteEventsByID

(

drone_id:"123123123123213"

drone_type:"123"

all:true

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| drone_id | String | ⽆⼈机ID |
| drone_type | String | ⽆⼈机类型 |

| Name | Type | Description |
| --- | --- | --- |
| all | Boolean | 是否删除 |

Response Examples

Success

{

"data": {

"deleteEventsByID": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"al\" on field \"deleteEventsByID\" of type

\"Mutation\". Did you mean \"all\"?",

"locations": [

{

"line": 7, "column": 5

}

]

}

]

}

DeleteWhitelist 移除⽩名单⽆⼈机

Method：POST

Request example

mutation

{

deleteWhitelist

(

id:"3asdasdcz2qwe1"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | ⽆⼈机id |

Response Examples

Success

{

"data": {

"deleteWhitelist": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"i\" on field \"deleteWhitelist\" of type

\"Mutation\". Did you mean \"id\"?",

"locations": [

{

"line": 5, "column": 3

}

]

}

]

}

DetectingBands 检测频段开关

Method：POST

Request example

mutation

{

detectingBands

(

band:"band24" enable:false

) {

Boost24 band12 band14 band18 band24 band4 band58

band9

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| band | String | 检测频段 |
| enable | boolean | 开启或关闭 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| Boost24 | Int | 0：不可⽤ 1：禁⽤ 2：启⽤ |
| band12 | Int | 1.2Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band14 | Int | 1.4Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band18 | Int | 1.8Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band24 | Int | 2.4Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band4 | Int | 443Mhz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band58 | Int | 5.8Ghz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |
| band9 | Int | 900Mhz 频段的状态，0:不可⽤，1:禁⽤，2:启⽤。 |

Response Examples

Success

{

"data": {

"detectingBands": { "Boost24": 0, "band12": 0, "band14": 2, "band18": 0, "band24": 1, "band4": 2, "band58": 2, "band9": 2

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"band\" on type \"DetectingBands\". Did you mean

\"band14\", \"band9\", \"band12\", \"band4\", or \"band24\"?",

"locations": [

{

"line": 15, "column": 7

}

]

}

]

}

Device 设备设置

Method：POST

Request example

mutation

{

device

(

class:"jammer" config:"

{\"id\":\"1\",\"group\":\"Bt\",\"mac\":\"48:b0:2d:51:2f:ee\",\"ip\":\"192.178.1.88/24\",\"sn \":\"SFFFFFFFF7430001OC1H\",\"capabilities\":[true,false,true,false,false],\"geo_location\": {\"lat\":40.041096,\"lng\":116.367195},\"location_locked\":true,\"pa_cfg\":[{\"range_mhz\": [0,0],\"max_dbm\":0,\"set_dbm\":0},{\"range_mhz\":[0,0],\"max_dbm\":0,\"set_dbm\":0}, {\"range_mhz\":[0,0],\"max_dbm\":0,\"set_dbm\":0},{\"range_mhz\": [0,0],\"max_dbm\":0,\"set_dbm\":0},{\"range_mhz\":[0,0],\"max_dbm\":0,\"set_dbm\":0}, {\"range_mhz\":[0,0],\"max_dbm\":0,\"set_dbm\":0},{\"range_mhz\": [0,0],\"max_dbm\":0,\"set_dbm\":0},{\"range_mhz\":[0,0],\"max_dbm\":0,\"set_dbm\":0}, {\"range_mhz\":[0,0],\"max_dbm\":0,\"set_dbm\":0},{\"range_mhz\": [0,0],\"max_dbm\":0,\"set_dbm\":0}],\"pa_power\": [],\"attack_wifi\":\"\",\"azimuth_start\":0,\"azimuth_end\":360,\"elevation_start\":-60,\"el evation_end\":90,\"work_mode\":0,\"jammer_type\":-1,\"heading\":90,\"azimuth_init\":0,\"elev ation_init\":0,\"azimuth_attack_range\":60,\"elevation_attack_range\":45,\"dir_delta\":2,\"v ersion\":\"2024.03.19|dummy\",\"build_time\":\"2024-03- 19T00:51:53Z\",\"git_hash\":\"6c68418d99c629c6e0eb0becd3ccf14618f68f33\"}"

id:"1"

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| id | String | 设备id |
| class | String | 设备类别 |
| config | jsonStrType | 设备 json 对象中的配置 |

Response Examples

Success

{

"data": {

"device": null

}

}

Failure

{

"data": {

"device": null

},

"errors": [

{

"message": "invalid jammer id", "locations": [

{

"line": 3, "column": 3

}

],

"path": [

"device"

]

}

]

}

DeviceTest ⾃检测

Method：POST

Request example

mutation

{

deviceTest

(

enable:false

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| enable | String | 启动（启⽤ = true）或停⽌（启⽤ = false）测试 |

Response Examples

Success

{

"data": {

"deviceTest": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"enabl\" on field \"deviceTest\" of type

\"Mutation\". Did you mean \"enable\"?",

"locations": [

{

"line": 5,

"column": 3

}

]

}

]

}

Dfgun 察打枪Method：POST Request example

mutation

{

dfgun

(

lan_type:123 mode:123 droneid:"123" attack_freq_low:123

attack_freq_high:123 cps_calib:false

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| lan_type | Int | 切换语⾔，0 _英语，1 _ 中⽂ |
| mode | Int | 0_ 正常，1_ 锁定 |
| droneid | String | ⽆⼈机ID |
| attack_freq_low | Int | 千赫兹，⼤于 300_000 |
| attack_freq_high | Int | 千赫兹，⼩于 2000_000 |
| cps_calib | Boolean | 罗盘校准 |

Response Examples

Success

{

"data": {

"deviceTest": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"enabl\" on field \"deviceTest\" of type

\"Mutation\". Did you mean \"enable\"?",

"locations": [

{

"line": 5, "column": 3

}

]

}

]

}

DhcpEnable 开启/关闭动态主机配置服务

Method：POST

Request example

mutation{ dhcpEnable (on:true)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| on | Boolean | 开关：true--开启，false--关闭 |

Response Examples

Success

{

"data": {

"dhcpEnable": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Argument \"on\" has invalid value true1.\nExpected type \"Boolean\", found

true1.",

"locations": [

{

"line": 3, "column": 7

}

]

}

]

}

DirectionalWBAttack 定向打击

Method：POST

Request example

mutation

{

directionalWBAttack

(

status:true band:"band12" gain:100 droneId:"123"

) {

band12 band14 band15 band18 band24 band4 band58 band9

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| status | Boolean | 是否开启 |
| band | String | 频段 |
| gain | Int | 增幅 |
| droneId | String | ⽆⼈机ID |

Responses

| Name | Type | Description |
| --- | --- | --- |
| band12 | Int | 频段 1.2G，0:不可⽤，1:空闲，其他:繁忙 |
| band14 | Int | 频段 1.4G，0:不可⽤，1:空闲，其他:繁忙 |
| band15 | Int | 频段 1.5G，0:不可⽤，1:空闲，其他:繁忙 |
| band18 | Int | 频段 1.8G，0:不可⽤，1:空闲，其他:繁忙 |

| Name | Type | Description |
| --- | --- | --- |
| band24 | Int | 频段 2.4G，0:不可⽤，1:空闲，其他:繁忙 |
| band4 | Int | 频段 433M，0:不可⽤，1:空闲，其他:繁忙 |
| band58 | Int | 频段 5.8G，0:不可⽤，1:空闲，其他:繁忙 |
| band9 | Int | 频段 900M，0:不可⽤，1:空闲，其他:繁忙 |

Response Examples

Success

{

"data": {

"directionalWBAttack": { "band12": 1, "band14": 1, "band15": 1, "band18": 1, "band24": 1,

"band4": 1, "band58": 1, "band9": 1

}

}

}

DroneAdsB 开启或关闭ADS-B

Method：POST

Request example

mutation

{

droneAdsB

(

switch:true

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| switch | Boolean | 是否开启 |

Response Examples

Success

{

"data": {

"droneAdsB": true

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"switc\" on field \"droneAdsB\" of type

\"Mutation\". Did you mean \"switch\"?",

"locations": [

{

"line": 5,

"column": 3

}

]

}

]

}

DroneProfileAlias 为指定⽆⼈机类型添加或删除别名

Method：POST

Request example

mutation{

droneProfileAlias( action:"add" id:"123" drone_type:"123" alias:"123" visible:false

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| action | String | 操作，可以添加或删除 |
| id | String | ⽆⼈机ID |

| Name | Type | Description |
| --- | --- | --- |
| drone_type | String | ⽆⼈机类型 |
| alias | String | ⾃定义⽆⼈机别名 |
| description | String | 机型备注 |
| visible | Boolean | 显⽰⽆⼈机为真，隐藏为假，默认为真 |

Response Examples

Success

{

"data": {

"droneProfileAlias": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Argument \"visible\" has invalid value falsed.\nExpected type \"Boolean\",

found falsed.",

"locations": [

{

"line": 7, "column": 13

}

]

}

]

}

DroneProfilePattern 添加或删除⽆⼈机正则表达式模式

Method：POST

Request example

mutation{

droneProfilePattern( new_pattern:"123" drone_type:"DJI MavicAir2" description:"123" visible:false

action:"add" pattern:"123"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| new_pattern | String | 如果操作为“add”并且此参数不为null，则旧模式将被此新模式替换 |
| drone_type | String | 定制⽆⼈机类型 |
| description | String | ⽆⼈机类型和模式描述 |
| visible | Boolean | 显⽰⽆⼈机为真，隐藏为假，默认为真 |
| action | String | 操作，可以添加或删除 |
| pattern | String | ⽆⼈机的模式 |

Response Examples

Success

{

"data": {

"droneProfilePattern": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Argument \"visible\" has invalid value falsed.\nExpected type \"Boolean\",

found falsed.",

"locations": [

{

"line": 6, "column": 13

}

]

}

]

}

DroneSub 订阅⽆⼈机截图信息（图传）

Method：POST

Request example

mutation

{

droneSub

(

id:"false" attribute:"123" enable:false

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | ⽆⼈机ID |
| attribute | String | ⽆⼈机的属性，⽀持列表： “截图” |
| enable | Boolean | 启⽤或禁⽤订阅 |

Response Examples

Success

{

"data": {

"droneSub": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"enabl\" on field \"droneSub\" of type

\"Mutation\". Did you mean \"enable\"?",

"locations": [

{

"line": 7,

"column": 5

}

]

}

]

}

Drone_profile 修改⽆⼈机属性

Method：POST

Request example

mutation

{

drone_profile

(

visible:false id:"9c2ba6791fd2" drone_link_id:"false" drone_type:"00001" drone_description:"zxc"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | ⽆⼈机ID |
| drone_link_id | String | ⽆⼈机关联ID |
| drone_type | String | ⽆⼈机类型 |
| drone_description | String | ⽆⼈机详细描述 |

Response Examples

Success

{

"data": {

"drone_profile": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"drone_descriptio\" on field \"drone_profile\" of

type \"Mutation\". Did you mean \"drone_description\"?",

"locations": [

{

"line": 9, "column": 5

}

]

}

]

}

Gfsk 通⽤检测器

Method：POST

Request example

mutation {

gfsk( cmd:"Add", data:"

{\"bandwidth\":2,\"protocol\":\"A7106\",\"chan_lis\":\"12312321\",\"bitrate\":1}"

option: "{\"payload_field_len\":3,\"fec\":false,\"crc\":false}"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| cmd | String | add_ 新增，delete_ 删除，modify_修改 |
| data | jsonStrType | 协议信息 |
| option | jsonStrType | 协议剩余信息（如：fec_信道编码、CRC校验等） |

Response Examples

Success

{

"data": {

"gfsk": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"drone_descriptio\" on field \"drone_profile\" of

type \"Mutation\". Did you mean \"drone_description\"?",

"locations": [

{

"line": 9, "column": 5

}

]

}

]

}

GvsControl 控制光电服务（⾃动校准服务）

Method：POST

Request example

mutation

{

gvsControl

(

id:"false" obj:"123" type:"false" action:"123" param:123

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| obj | String | add_ 新增，delete_ 删除，modify_修改 |
| type | String | 操作类型，如：autocali |

| Name | Type | Description |
| --- | --- | --- |
| action | String | 操作动作，如：startAutocali |
| id | String | 设备ID |
| param | List | 下发服务需要的附加参数 |

Response Examples

Success

{

"data": {

"drone_profile": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"drone_descriptio\" on field \"drone_profile\" of

type \"Mutation\". Did you mean \"drone_description\"?",

"locations": [

{

"line": 9,

"column": 5

}

]

}

]

}

MarkFalseAlarm 标记误报（光电）

Method：POST

Request example

mutation {

markFalseAlarm

(sequence: [1])

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| sequence | [Int] | 要标记的事件序列数组 |

Response Examples

Success

{

"data": {

"markFalseAlarm": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Argument \"sequence\" has invalid value asd.\nExpected type \"Int\", found

asd.",

"locations": [

{

"line": 3, "column": 16

}

]

}

]

}

MarkFalseAlarmByID 按ID标记误报（光电）

Method：POST

Request example

mutation{ markFalseAlarmByID (

all:false drone_type:"123" drone_id:"123"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| all | Boolean | 是否删除所有事件 |
| drone_type | String | 查询⽆⼈机类型 |
| drone_id | String | 查询⽆⼈机ID |

Response Examples

Success

{

"data": {

"markFalseAlarmByID": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Argument \"all\" has invalid value 123.\nExpected type \"Boolean\", found

123.",

"locations": [

{

"line": 4,

"column": 9

}

]

}

]

}

Node 节点Method：POST Request example

mutation

{

node

(

name:"自行车5" info:"自行车" cmd:"add"

id:"LZAac7c57d7-6096-49ce-bfa0-ef1f761dfff01"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| cmd | String | 操作：添加、删除、修改、采⽤、丢弃、重置、⽇志清理、⽇志上传 |
| id | String | 节点ID |
| name | String | 节点名称 |
| info | String | 描述信息 |

Response Examples

Success

{

"data": {

"node": true

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"i\" on field \"node\" of type \"Mutation\". Did

you mean \"id\"?",

"locations": [

{

"line": 8, "column": 7

}

]

}

]

}

PowerMode 设置电源模式，仅适⽤于便携式设备

Method：POST

Request example

mutation{

powerMode

(mode:"balanced")

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| mode | String | “性能”：CRPC+DroneID+RemoteID，默认；“平衡”：DroneID+RemoteID； “省电”：RemoteID |

Response Examples

Success

{

"data": {

"powerMode": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Argument \"mode\" has invalid value balanced.\nExpected type \"String\",

found balanced.",

"locations": [

{

"line": 3, "column": 9

}

]

}

]

}

Remove_fake_drone 标记⽆⼈机为误报⽆⼈机（⽆⼈机列表）

Method：POST

Request example

mutation

{

remove_fake_drone

(

uuid:"eeb8ce4b-9633-4fdd-8ba3-5769057bbbc2"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| mutation | object | 操作 |
| remove_fake_drone | None | 误报 |
| uuid | String | ⽆⼈机UUID |

Response Examples

Success

{

"data": {

"remove_fake_drone": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"engineI\" on field \"rcap\" of type \"Mutation\".

Did you mean \"engineId\"?",

"locations": [

{

"line": 7, "column": 5

}

]

}

]

}

Reset 重启设备

Method：POST

Request example

mutation

{

reset

(

id:"LZAba14df4a-1ded-4bb5-b9da-fa4aac92abac1"

class:"auth" reason:"123"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | 设备id |
| class | String | 设备类别（如：controller-控制器） |
| reason | String | 重启原因 |

Response Examples

Success

{

"data": {

"reset": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"reaso\" on field \"reset\" of type \"Mutation\".

Did you mean \"reason\"?",

"locations": [

{

"line": 7,

"column": 5

}

]

}

]

}

RollbackPatch 切换或升级 UI 版本

Method：POST

Request example

mutation

{

rollbackPatch

(

class:"ui" version:0

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| class | String | 补丁类型，当前仅⽀持：控制器、引擎、⽤⼾界⾯ |
| version | Int | 0：默认版本，1：旧版本。 |

Response Examples

Success

{

"data": {

"rollbackPatch": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"versio\" on field \"reset\" of type \"Mutation\".

Did you mean \"version\"?",

"locations": [

{

"line": 7, "column": 5

}

]

}

]

}

SetGeoLocation 设置设备位置（⾮持久化）

Method：POST

Request example

mutation{ setGeoLocation (

lat:12.1212,

lng:21.1212

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| lat | Float | 纬度 |
| lng | Float | 经度 |

Response Examples

Success

{

"data": {

"setGeoLocation": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Argument \"lng\" has invalid value \"21.1212\".\nExpected type \"Float\",

found \"21.1212\".",

"locations": [

{

"line": 5, "column": 9

}

]

}

]

}

Set_gvs_conf 修改光电配置

Method：POST

Request example

mutation

{

set_gvs_conf

(

cont:"123" id:"123"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| cont | String | 配置信息 |
| id | String | 设备ID |

Response Examples

Success

{

"data": {

"set_gvs_conf": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"i\" on field \"set_gvs_conf\" of type

\"Mutation\". Did you mean \"id\"?",

"locations": [

{

"line": 6, "column": 5

}

]

}

]

}

Set_sensor_mode 设置传感器模式

Method：POST

Request example

mutation

{

set_sensor_mode

(

mode:"DirectionFinding"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| mode | String | DetectionOnly_ 检测、DirectionFinding_ 定向、Detection+DF_ 检测+定向、SpectrumScanner_ 遗弃 |

Response Examples

Success

{

"data": {

"set_sensor_mode": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"mod\" on field \"set_sensor_mode\" of type

\"Mutation\". Did you mean \"mode\"?",

"locations": [

{

"line": 5, "column": 3

}

]

}

]

}

Set_vsg_license 设置光电授权

Method：POST

Request example

mutation

{

set_vsg_license

(

id:"1" license:"12ASJDHLAKSDAKJLBkjksjgdbkj……

sdbfalksdhnaslkdnasdaskfdjgabsdkjlbasdkjasbdoiwquheiqouweqoqiwhdqowiehnksjaldajkmdnasd3"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | 设备ID |
| license | String | 设备许可证 |

Response Examples

Success

{

"data": {

"set_vsg_license": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"mod\" on field \"set_sensor_mode\" of type

\"Mutation\". Did you mean \"mode\"?",

"locations": [

{

"line": 5,

"column": 3

}

]

}

]

}

SigParse 信号分析

Method：POST

Request example

mutation

{

sigParse

(

short:false sensorId:"1" bands:["band9"] brief:false extFreq:0 antenna:0

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| sensorId | String | 传感器ID |
| bands | [String] | 信号解析的频段 |
| extFreq | Int | 信号解析的额外频率（Mhz）。0 为空 |
| short | Boolean | 持续时间设置。false（正常）：21ms，ture（短）：10ms |
| brief | Boolean | true：打开 false：关闭 |
| antenna | Int | 天线接受的信号来⾃：[0-7] |

Response Examples

Success

{

"data": {

"sigParse": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"brie\" on field \"sigParse\" of type

\"Mutation\". Did you mean \"brief\"?",

"locations": [

{

"line": 9, "column": 5

}

]

}

]

}

signalCap 信号抓取

Method：POST

Request example

mutation

{

signalCap

(

msg:

" {\"sigCap_drone\":\"111\",\"sigCap_distance\":\"222\",\"sigCap_height\":\"11\",\"sigCap_loca tion\":\"shx\",\"sigCap_comment\":\"无备注\"}"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| msg | String | 信号抓取记录信息 |

Response Examples

Success

{

"data": {

"sigParse": null

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"brie\" on field \"sigParse\" of type

\"Mutation\". Did you mean \"brief\"?",

"locations": [

{

"line": 9,

"column": 5

}

]

}

]

}

SignalRec 信号证据记录

Method：POST

Request example

mutation

{

signalRec

(

id:"4418" cmd:"123" droneTypes:"123"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | String | 引擎ID |
| cmd | String | 信号记录命令 |
| droneType | String | ⽤于信号记录的⽆⼈机类型 |

Response Examples

Success

{

"data": {

"signalRec": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"droneType\" on field \"signalRec\" of type

\"Mutation\". Did you mean \"droneTypes\"?",

"locations": [

{

"line": 7, "column": 5

}

]

}

]

}

Spd 频谱Method：POST Request example

mutation

{

spd

(

sensor_id:"1" req_id:1 freq_start:30 freq_stop:6000 span_points:1

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| freq_start | Int | 频谱跨度的起始频率，单位 Kh |
| freq_stop | Int | 频谱跨度的结束频率，单位 Khz |
| span_points | Int | 频谱扫描点数 |
| action | String | "add"添加or "update"修改 or "remove"_删除 |
| sensor_id | String | SPD 传感器的 ID |
| req_id | Int | 此 SPD 会话的请求 ID |

Response Examples

Success

{

"data": {

"signalRec": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"droneType\" on field \"signalRec\" of type

\"Mutation\". Did you mean \"droneTypes\"?",

"locations": [

{

"line": 7, "column": 5

}

]

}

]

}

Spoofer_switch 诱骗设置

Method：POST

Request example

mutation

{

spoofer_switch

(

id:"3" action:1 timeout:1 direction:-1

)

{

switch

}

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| id | String | 诱骗器ID，空表⽰⾃动选择 |
| action | Int | 屏蔽开关，0:关闭，1:打开，2:迫降，3:驱离 |
| timeout | Int | 开启 诱骗器的超时（秒），0 表⽰⽆限超时 |
| direction | Int | 驱逐欺骗的⽅向，-1 表⽰⾃动选择 |

Responses

| Name | Type | Descriptio |
| --- | --- | --- |
| switch | Int | 开关状态，0:关闭，1:打开，2:迫降，3:驱离。 |

Response Examples

Success

{

"data": {

"spoofer_switch": {

"switch": 0

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"switc\" on type \"spoofStatus\". Did you mean

\"switch\"?",

"locations": [

{

"line": 11, "column": 5

}

]

}

]

}

StageReset 定向复位

Method：POST

Request example

mutation

{

stageReset

(

direction:0 elevation:0 track_enable:true id:"123"

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| elevation | Float | 垂直范围（-60<=value<=90）默认0 |
| track_enable | Boolean | true -- 复位后继续跟踪已发现的⽆⼈机 false -- 复位后停⽌追踪⽆⼈机 |
| id | String | ⼲扰器ID |
| direction | Float | ⽔平范围（0<=value<=360）默认0 |

Response Examples

Success

{

"data": {

"stageReset": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Cannot query field \"stageReset\" on type \"Query\".", "locations": [

{

"line": 2, "column": 2

}

]

}

]

}

Tdoa_track TDOA跟踪

Method：POST

Request example

mutation

{

tdoa_track

(

id:"304a262b3a59" enable:false

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| id | Float | ⽆⼈机的 ID |
| enable | Boolean | 如果已设置，则开始对⽆⼈机进⾏ tdoa 跟踪 |

Response Examples

Success

{

"data": {

"tdoa_track": null

}

}

Failure

{

"data": {

"tdoa_track": null

},

"errors": [

{

"message": "The drone doesn't exist",

"locations": [

{

"line": 3,

"column": 3

}

],

"path": [

"tdoa_track"

]

}

]

}

Time 设置设备系统时间

Method：POST

Request example

mutation

{

time

(

setTime:1710777670

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| setTime | Int | 设置时间（时间戳) |

Response Examples

Success

{

"data": {

"time": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"setTim\" on field \"time\" of type \"Mutation\".

Did you mean \"setTime\"?",

"locations": [

{

"line": 5, "column": 3

}

]

}

]

}

TimeZone 设置设备时区

Method：POST

Request example

mutation

{

timeZone

(

setTimeZone:"Africa/Harare" auto:false

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| setTimeZone | String | 设置时区 |
| auto | Boolean | True 表⽰根据 GPS 定位⾃动配置，false 表⽰⽤⼾⼿动配置 |

Response Examples

Success

{

"data": {

"timeZone": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"setTimeZon\" on field \"timeZone\" of type

\"Mutation\". Did you mean \"setTimeZone\"?",

"locations": [

{

"line": 5, "column": 3

}

]

}

]

}

Tracking ⽆⼈机视觉跟踪

Method：POST

Request example

mutation

{

tracking

(

id:"IG2NZMvayzTaMvDwRXN63cd1c6c68"

uuid:"304a262b3a59"

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| id | String | 设备ID |
| uuid | Int | 设置后，跟踪⽆⼈机 未设置，则取消对⽆⼈机的跟踪 |

Response Examples

Success

{

"data": {

"tracking": null

}

}

Failure

{

"data": {

"tracking": null

},

"errors": [

{

"message": "invalid UUID length: 19",

"locations": [

{

"line": 3,

"column": 3

}

],

"path": [

"tracking"

]

}

]

}

UnknownWifi 未知WIFI

Method：POST

Request example

mutation

{

unknownWifi

(

mac:"60601f5fb31f" tag:"drone"

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| mac | String | ⽆⼈机macID |
| tag | String | 设置当前⽆⼈机的标签：“热点”、“⽆⼈机”、"未知 |

Response Examples

Success

mutation

{

unknownWifi

(

mac:"60601f5fb31f" tag:"drone"

)

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"ta\" on field \"unknownWifi\" of type

\"Mutation\". Did you mean \"tag\"?",

"locations": [

{

"line": 6, "column": 5

}

]

}

]

}

UpgradePatch 补丁升级

Method：POST

Request example

mutation

{

upgradePatch

}

Parameter Description

None

Response Examples

Success

{

"data": {

"upgradePatch": null

}

}

Failure

{

"data": {

"upgradePatch": null

},

"errors": [

{

"message": "the base version is inconsistent, expect version: 2024.10.18

,reality version: 3.14.0",

"locations": [

{

"line": 3, "column": 3

}

],

"path": [

"upgradePatch"

]

}

]

}

UpgradeTo 版本升级

Method：POST

Request example

mutation

{

upgradeTo

(

versionName:"v2024.03.11" upgradeBy:"shengji"

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| versionName | String | 已存在的本地版本名称 |
| upgradeBy | String | 升级 |

Response Examples

Success

成功后重启，返回连接失败

错误：socket hang up

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"upgradeB\" on field \"upgradeTo\" of type

\"Mutation\". Did you mean \"upgradeBy\"?",

"locations": [

{

"line": 6,

"column": 5

}

]

}

]

}

VersionPackage 版本包

Method：POST

Request example

mutation

{

versionPackage

(

filename:"v1.2.3"

password:"zxcasdqeqgsfge3453……5sdgvdsfg32er"

)

}

Parameter Description

Requests

| Name | Type | Descriptio |
| --- | --- | --- |
| filename | String | 上传更新包的⽂件名 |
| password | String | 上传更新包的密码 |

Response Examples

Success

{

"data": {

"versionPackage": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"passwor\" on field \"versionPackage\" of type

\"Mutation\". Did you mean \"password\"?",

"locations": [

{

"line": 6,

"column": 5

}

]

}

]

}

Wideband_attack 宽频⼲扰

Method：POST

Request example

mutation

{

wideband_attack

(

band: "band24" wb_status:true gain:50 direction:-1 elevation:5 directional:false jammer_list:["[1]"]

) { band12 band14 band15 band18 band24 band4 band58 band9

}

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| band | String | 指定频段 |
| wb_status | Boolean | 宽带⼲扰器的状态，状态为真时⼲扰器打开，状态为假时⼲扰器关闭 |
| gain | Int | 宽带⼲扰器的增益 |
| direction | Float | 宽带⼲扰器的发展⽅向 |
| elevation | Float | 宽带⼲扰器的提升 |

| Name | Type | Description |
| --- | --- | --- |
| directional | String | 攻击使⽤⼲扰器列表 |
| jammer_list | list | 频段列表 |

Responses

| Name | Type | Description |
| --- | --- | --- |
| band12 | Int | 频段状态 1.2G，0:不可⽤，1:空闲，其他:繁忙。 |
| band14 | Int | 频段状态 1.4G，0:不可⽤，1:空闲，其他:繁忙。 |
| band15 | Int | 频段状态 1.5G，0:不可⽤，1:空闲，其他:繁忙。 |
| band18 | Int | 频段状态 1.8G，0:不可⽤，1:空闲，其他:繁忙。 |
| band24 | Int | 频段状态 2.4G，0:不可⽤，1:空闲，其他:繁忙。 |
| band4 | Int | 频段状态 433M，0:不可⽤，1:空闲，其他:繁忙。 |
| band58 | Int | 频段状态 5.8G，0:不可⽤，1:空闲，其他:繁忙。 |
| band9 | Int | 频段状态 900M，0:不可⽤，1:空闲，其他:繁忙。 |

Response Examples

Success

{

"data": {

"wideband_attack": { "band12": 1, "band14": 1, "band15": 1, "band18": 1, "band24": 1, "band4": 1, "band58": 1, "band9": 1

}

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"jammer_lis\" on field \"wideband_attack\" of type

\"Mutation\". Did you mean \"jammer_list\"?",

"locations": [

{

"line": 11, "column": 5

}

]

}

]

}

WifiBgLearning wifi背景学习开关

Method：POST

Request example

mutation

{

wifiBgLearning

(

enable:false reset:false

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| enable | Boolean | true 表⽰开始学习，false 表⽰停⽌学习 |
| reset | Boolean | 为真，删除当前数据并重新创建 |

Response Examples

Success

{

"data": {

"wifiBgLearning": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"rese\" on field \"wifiBgLearning\" of type

\"Mutation\". Did you mean \"reset\"?",

"locations": [

{

"line": 6,

"column": 5

}

]

}

]

}

WithdrawFalseAlarm 还原误报事件（光电）

Method：POST

Request example

mutation {

withdrawFalseAlarm(

drone_type: "DJI Mavic"

drone_id: "21d5b875-e839-4d33-bcd3-1d4f013e30b4"

)

}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| drone_type | String | ⽆⼈机类型 |
| drone_id | String | ⽆⼈机ID |

Response Examples

Success

{

"data": {

"wifiBgLearning": null

}

}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"rese\" on field \"wifiBgLearning\" of type

\"Mutation\". Did you mean \"reset\"?",

"locations": [

{

"line": 6,

"column": 5

}

]

}

]

}

Subscription 订阅

接⼝：Websocket

Url：wss://192.168.100.100/sub/subscriptions

Drone ⽆⼈机Method：POST Request example

{"id":"2","type":"add","payload":{"query":"subscription{\n   drone {\n	id,\n

name,\n	description,\n	image,\n	secret,\n	state,\n	direction,\n

distance,\n	whitelisted,\n	blacklisted,\n	can_attack,\n	can_takeover,\n

can_ctrl_landing,\n	in_ada,\n	speed,\n	attacking,\n	ctrl_landing,\n

attacking_ttl,\n	created_time,\n	deleted_time,\n	lastseen_time,\n

lastseen,\n	longitude,\n	latitude,\n	height,\n	seen_sensor{\n

snr_dB,\n	sensor_id,\n	detected_freq_khz,\n	signal_dbm,\n

port\n	},\n	attack_bands,\n	attack_type,\n	tracing{\n	lastlen\n

origin{\n	lat\n	lng\n	},points\n	}\n

localization{\n	lat,\n	lng\n	},\n	initial_location{\n

lat,\n	lng\n	},\n	rc_location{\n	lat,\n	lng\n	},\n

link_id,\n	has_duplicate,\n	jamming_conflicts,\n	confirmed,\n

tracking_video,\n	has_screenshot,\n	directional_attack_state\n   }\n

}","extensions":{},"operationName":null,"variables":{}}}

Parameter Description

| Name | Type | Description |
| --- | --- | --- |
| id | String | 消息ID |
| type | String | 类型 |
| payload | String | 有效载荷 |
| query | Boolean | 查询 |
| subscription | None | 订阅 |

| Name | Type | Description |
| --- | --- | --- |
| query | String | None |
| drone | None | ⽆⼈机信息 |
| id | String | 检测到的⽆⼈机id |
| altitude | Float | 海拔 |
| attack_bands | Int | ⽆⼈机信号 |
| attack_type | String | ⽆⼈机打击类型（Wideband宽频、Smart_CRPC打击、Smart-II精准打击、null） |
| attacking | Boolean | 是否在精准打击（true是，false否） |
| attacking_ttl | Int | 精准打击时⻓（倒计时） |
| blacklisted | Boolean | ⽆⼈机是否被列⼊⿊名单 |
| can_attack | Boolean | ⽆⼈机能否被精准打击（true、false） |
| can_ctrl_landing | Boolean | ⽆⼈机能否受控迫降 |
| can_takeover | Boolean | ⽆⼈机是否可以被接管 |
| confirmed | Boolean | 确定是⽆⼈机或只是可疑 |
| created_time | DateTime | ⾸次检测到的时间 |
| ctrl_landing | Boolean | ⽆⼈机正在受控降落 |
| deleted_time | DateTime | 消失时间 |
| description | String | ⽆⼈机描述 |
| direction | Float | ⽆⼈机⻆度（365表⽰⽆效） |
| directional_attack_state | Int | 定向打击状态（0空闲、1追踪、2攻击） |
| distance | Float | ⽆⼈机到设备中⼼点的距离 |
| has_duplicate | Boolean | ⽆⼈机是否隐藏 |
| has_screenshot | Boolean | 图传 |
| height | Float | ⽆⼈机⾼度 |
| id | String | ⽆⼈机ID |
| image | String | ⽆⼈机图⽚ |
| in_ada | Int | ⽆⼈机在防空区（0不可⽤、1不在、2预警区、3防御区） |
| initial_location{lat,lng} | String | ⽆⼈机初始位置，210表⽰⽆效 |
| jamming_conflicts | String | 误打列表 |

| Name | Type | Description |
| --- | --- | --- |
| lastseen | String | 最后发现的时间 |
| lastseen_time | DateTime | 最后一次检测到的时间 |
| latitude | String | 纬度 |
| link_id | Float | ⽆⼈机关联ID |
| localization{lat,lng} | {Float} | 对⽆⼈机进⾏跟踪（已弃⽤） |
| longitude | Float | 经度 |
| name | String | ⽆⼈机机型或别名 |
| rc_location{lat,lng} | GeoPoint | ⻜⼿位置（遥控器 210表⽰⽆效）{经纬度} |
| screenshot | String | ⽆⼈机最新截图（JPG格式） |
| seen_sensor | SignalObject | 传感器检测到⽆⼈机 |
| bandwidth_khz | Int | 信号带宽 |
| detected_freq_khz | Int | 信号频点 |
| noise_dbm | Int | 天线的射频噪声功率，分⻉差为一毫⽡ |
| port | String | 检测端⼝ |
| sensor_id | String | 检测的传感器ID |
| signal_dbm | Int | 信号⼤⼩ |
| snr_dB | Float | 信噪⽐ |
| speed | Float | ⽆⼈机速度（m/s） |
| state | String | ⽆⼈机状态（attacking打击、danger危险、Smart_打击类型） |
| tracing{lastlen} | String | ⽆⼈机轨迹点 |
| from | Int | 从 |
| lastlen | Int | 最后一个点的⻓度 |
| origin{lat,lng} | Float | 原地理位置 |
| points | [Float] | 追踪点 |
| tracking_video | String | 跟踪视频⽂件 |
| whitelisted | Boolean | 是否加⼊⽩名单 |

Response Examples

Success

{

"type": "data", "id": "2", "payload": {

"drone": [

{

"attack_bands": [], "attack_type": "null", "attacking": false, "attacking_ttl": 0, "blacklisted": false, "can_attack": false, "can_ctrl_landing": false, "can_takeover": false, "can_tdoa": false, "confirmed": false,

"created_time": "2024-10-21T13:51:41.132761021+08:00", "ctrl_landing": false,

"deleted_time": "0001-01-01T00:00:00Z",

"description": "", "direction": 365,

"directional_attack_state": 0, "distance": 0,

"has_duplicate": false, "has_screenshot": false, "height": 0,

"id": "60601f40e348",

"image": "dimages/dji_p3_rc.png", "in_ada": 0,

"initial_location": {

"lat": 210, "lng": 210

},

"jamming_conflicts": [], "lastseen": null,

"lastseen_time": "2024-10-21T13:51:56.131997501+08:00", "latitude": 0,

"link_id": "", "localization": null, "longitude": 0,

"name": "DJI Phantom3_RC", "rc_location": {

"lat": 210, "lng": 210

},

"secret": "", "seen_sensor": [

{

"detected_freq_khz": 5748000, "port": "rf",

"sensor_id": "2", "signal_dbm": 54, "snr_dB": -17.173069

}

],

"speed": 0, "state": "danger",

"tdoa_tracking": false,

"tracing": { "lastlen": 0, "origin": {

"lat": 0, "lng": 0

},

"points": []

},

"tracking_video": "", "whitelisted": false

},

……, {

"attack_bands": [

2400, 5800

],

"attack_type": "Smart-II", "attacking": false, "attacking_ttl": 0, "blacklisted": false, "can_attack": true, "can_ctrl_landing": false, "can_takeover": false, "can_tdoa": false, "confirmed": true,

"created_time": "2024-10-21T12:17:56.141919037+08:00", "ctrl_landing": false,

"deleted_time": "0001-01-01T00:00:00Z", "description": "zxc",

"direction": 365, "directional_attack_state": 0, "distance": 0,

"has_duplicate": false, "has_screenshot": false, "height": 0,

"id": "304a262b3a59",

"image": "dimages/Feipai_6k_pro.png", "in_ada": 0,

"initial_location": {

"lat": 210, "lng": 210

},

"jamming_conflicts": [], "lastseen": null,

"lastseen_time": "2024-10-21T13:51:56.132191293+08:00",

"latitude": 0, "link_id": "false",

"localization": null, "longitude": 0, "name": "00001", "rc_location": {

"lat": 210,

"lng": 210

},

"secret": "",

"seen_sensor": [

{

"detected_freq_khz": 5745000, "port": "wifi",

"sensor_id": "1", "signal_dbm": 54, "snr_dB": 13.370029

}

],

"speed": 0, "state": "danger",

"tdoa_tracking": false, "tracing": {

"lastlen": 0, "origin": {

"lat": 0, "lng": 0

},

"points": []

},

"tracking_video": "", "whitelisted": false

}

]

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"rese\" on field \"wifiBgLearning\" of type

\"Mutation\". Did you mean \"reset\"?",

"locations": [

{

"line": 6,

"column": 5

}

]

}

]

}

Message 后台消息

Request example

{"id":"2","type":"sub","payload":{"query":"subscription{\n   message{\n	msg_type\n

msg_key\n	info\n	source\n   }\n }","extensions":

{},"operationName":null,"variables":{}}}

Parameter Description

Requests

| Name | Type | Description |
| --- | --- | --- |
| id | object | 消息ID |
| type | String | 类型 |
| payload | Boolean | 有效载荷 |
| query | Boolean | 查询 |
| subscription | None | 订阅 |
| message | String | 从后台推送信息 |
| msg_type | None | 当前信息的类型 |
| msg_key | String | msg 重要关键字 |
| info | Float | 信息 |
| source | Int | 当前信息的来源 |

Response Examples

Success

{

"id": "2", "type": "sub", "payload": {

"query": "subscription{\n

message{\n

msg_type\n

msg_key\n

info\n

source\n   }\n }",

"extensions": {}, "operationName": null, "variables": {}

}

}

Failure

{

"data": null,

"errors": [

{

"message": "Unknown argument \"rese\" on field \"wifiBgLearning\" of type

\"Mutation\". Did you mean \"reset\"?",

"locations": [

{

"line": 6,

"column": 5

}

]

}

]

}

Notify 通知消息

Request example

{"id":"2","type":"start","payload":{"query":"subscription{\n   notify{\n	id\n

extra\n	key\n	info\n	time\n	source\n	status\n   }\n

}","extensions":{},"operationName":null,"variables":{}}}

Parameter Description

| Name | Type | Description |
| --- | --- | --- |
| id | object | 消息ID |
| type | String | 类型 |
| payload | Boolean | 有效载荷 |
| query | Boolean | 查询 |
| subscription | None | 订阅 |
| notify | String | 通知 |
| id | String | ID |
| extra | String | 额外信息 |
| info | String | 信息 |
| key | Int | msg ID |
| time | String | 时间 |
| source | String | 当前通知的来源 |
| status | String | 通知状态 |

Response Examples

Success

{"type":"data","id":"1","payload":{"notify":

{"extra":"","id":"1","info":"","key":21,"source":"jammer","status":"start","time":"2024-10-

21 14:38:40"}}}

Failure

{

"data": null, "errors": [

{

"message": "Unknown argument \"rese\" on field \"wifiBgLearning\" of type

\"Mutation\". Did you mean \"reset\"?",

"locations": [

{

"line": 6, "column": 5

}

]

}

]

}